AI Mode

All

Videos

Forums

Shopping

More

can I deploy VOSS on EXOS configured 5320 for EN

Yes, you can deploy VOSS (Fabric Engine) on an Extreme Networks 5320 Seriesswitch, even if it is currently configured with EXOS (Switch Engine). The 5320 is part of Extreme's Universal Hardware line, which features "dual-persona" capability, allowing you to choose or change the operating system at any time.

Extreme Networks +1

Key Deployment Considerations

- Operating System Switch: You can change the OS during the initial boot sequence or from within the existing OS.

- Data Erasure: Changing the OS will reformat the storage device, clearing all existing data, configurations, and logs.

- Feature Support: While the 5320 supports VOSS for Fabric Engine deployments, it does not support vIST (Virtual Inter-Switch Trunking); if you require redundancy at the edge, you might need to use MLAG in EXOS instead.

- Stacking: Fabric Engine does not support stacking on the 5320; stacking is only available when running Switch Engine (EXOS).

Extreme Portal +3

How to Change the OS

- Boot Menu: During startup, press the space bar to access the Boot Menu and select "Change the switch OS to VOSS".

- CLI (from EXOS): If already running EXOS, it may prompt you during the initial setup wizard: "Would you like to change the switch OS to VOSS? [y/N/q]".

- Automation: You can use the ExtremeCloud IQ (XIQ) platform to automate the OS selection and load the desired image remotely.

Extreme Portal +5

Note: The Extended Temperature models (5320-24T-4X-XT and 5320-24T-24S-4XE-XT) only support the EXOS operating system and cannot be converted to VOSS.

Extreme Portal

Would you like the specific CLI commands for manual conversion or information on Fabric Engine licensing for this model?

8 sites

Extreme Networkshttps://documentation.extremenetworks.com

5320 Series Overview

Mar 1, 2026 — 5320 Series Overview * Universal Hardware. The switch comes with a dual-persona capability, allowing you to choose your Operating ...

Extreme Portalhttps://extreme-networks.my.site.com

Option To Change OS From EXOS To VOSS Not Available ...

Jan 8, 2024 — Resolution. Two Solutions: 1. Switch NOS Selection at Startup of EXOS. If the switch is powered up and the spacebar was not presse...

Extreme Networkshttps://www.extremenetworks.com

5320 Series - Extreme Networks

The 5320 comes with a dual-persona capability, allowing you to choose your Operating System (OS). Either the Switch Engine (EXOS) ...

Show all

I have an E2E home-lab already setup with two 5320 switches each with one AP3000 connected with four VLANs with two VLANs each in the two switches with one management vlan extra aside from the default vlan-1. Running EXOS. I want to configure this with VOSS.

To transition your home-lab from EXOS to VOSS (Fabric Engine), you must first convert the switch personality and then reconfigure your networking services using VOSS-specific Fabric Connect logic.

1. Switch OS Conversion

Changing the OS will wipe all existing EXOS configurations. You can perform this conversion via the CLI or the Bootloader.

- Via EXOS CLI:

  - Download the VOSS image file to a TFTP server or USB.

  - Run the command: download image <TFTP_IP> <VOSS_IMAGE_NAME>.voss.

  - Select "Yes" at the install prompt and then "c" to continue the NOS change.

- Via Bootloader: During startup, press the space bar to enter the boot menu and select "Change the switch OS to VOSS".

Extreme Portal +4

2. Configure Management VLAN

VOSS uses "Segmented Management" for out-of-band or in-band control.

Extreme Networks

- In-Band Management:

voss

vlan create <VLAN_ID> type port-mstprstp 0

interface vlan <VLAN_ID>

ip address <IP_ADDR> <SUBNET_MASK>

exit

Use code with caution.

- Default Management: On the 5320, the management port is pre-assigned to the mgmt VLAN.

Extreme Networks +2

3. Configure Fabric Connect (VSNs) for VLANs

In VOSS, instead of traditional VLAN tagging across trunks, you map VLANs to I-SIDs(Individual Service Identifiers) to create a Fabric.

- Step A: Create VLANs and map to I-SIDs
Assign each of your 4 VLANs a unique I-SID to allow them to span across your two switches.

voss

vlan create <VLAN_ID> name "VLAN_NAME" type port-mstprstp 0

vlan i-sid <VLAN_ID> <I-SID_VALUE>

Use code with caution.

- Step B: Configure AP3000 Ports (Fabric Attach)
Since you have an AP3000, use Fabric Attach (FA). This allows the AP to automatically request the correct VLAN/I-SID mappings from the switch.

voss

interface GigabitEthernet <PORT_TO_AP>

fa enable

auto-sense enable

exit

Use code with caution.

- Step C: Connect the Switches (NNI)
To link the two 5320s into a single Fabric, configure the connecting ports as Network-to-Network Interfaces (NNI).

voss

interface GigabitEthernet <LINK_PORT>

isis enable

isis spbm 1

no shutdown

Use code with caution.

Extreme Portal +3

4. Verification

Check your setup with these commands:

- show isis adj: Verify the link between your two switches.

- show fa assignment: Check if the AP3000 has successfully mapped its SSIDs to your VLANs.

Do you have the VOSS image file ready, or do you need assistance locating the correct firmware version for the 5320?

14 sites

Extreme Portalhttps://extreme-networks.my.site.com

Option To Change OS From EXOS To VOSS Not Available ...

Jan 8, 2024 — 2. Change Switch NOS from EXOS to VOSS using CLI * Switching the NOS personality to VOSS from EXOS uses the same process as perfor...

Extreme Networkshttps://documentation.extremenetworks.com

VLAN - VOSS User Guide - Extreme Networks

Oct 15, 2021 — Configure a Segmented Management Instance Using quick-config-mgmt Utility. Create a Segmented Management Instance. Delete a Segmen...

Extreme Networkshttps://documentation.extremenetworks.com

Changing the Network Operating System

May 1, 2024 — Making Your Initial Network Operating System Selection. You can make your initial selection of the operating system using: Extreme...

Show all

I have the VOSS image in XIQ I can deploy? Also, can you create a table with EXOS on left and VOSS on right for this migration on what I need to do thematically and in sequence, step by step?

Yes, you can deploy the VOSS image directly from ExtremeCloud IQ (XIQ). For a Universal Hardware switch like the 5320, you can select the device in XIQ and use the "Change OS" action to trigger the persona change. Once the switch reboots, it will download the VOSS image and transition to Fabric Engine.

Extreme Portal +2

Migration Sequence: EXOS to VOSS

Because VOSS uses Shortest Path Bridging (SPB/Fabric Connect) instead of traditional VLAN/Spanning Tree logic, the migration follows a different architectural theme.

| Sequence | Theme | EXOS (Switch Engine) Action | VOSS (Fabric Engine) Action |
| --- | --- | --- | --- |
| 1 | Personality Change | download image <url> (VOSS file) & select y to switch OS. | Switch reboots, wipes EXOS config, and starts as VOSS. |
| 2 | Management | configure vlan mgmt ipaddress <IP> | vlan create <ID> type port-mstprstp 0 followed by interface vlan <ID> for in-band IP. |
| 3 | Fabric Core (NNI) | create sharing (LAG) + configure vlan <name> add ports(Trunk) | Enable IS-IS on the physical links: isis enable, isis spbm 1. |
| 4 | VLAN Services | create vlan <VLAN_ID> | vlan create <VLAN_ID> + vlan i-sid <VLAN_ID> <I-SID>(Maps VLAN to Fabric). |
| 5 | AP Connectivity | configure vlan <ID> add ports <AP_Port> tagged | interface gigabitethernet <AP_Port> -> fa enable (Fabric Attach). |
| 6 | AP Automation | Manual VLAN assignment per port/SSID. | auto-sense enable; switch automatically assigns VLANs based on AP requests. |

Summary of Differences

- VLAN Persistence: In EXOS, you tag ports. In VOSS, you map a VLAN to an I-SID (a global service ID), and the Fabric handles the transport across switches automatically.

- Fabric Attach (FA): For your AP3000, VOSS is significantly more automated. By enabling Auto-sense and FA on the AP ports, the switch will dynamically create the necessary mappings when the AP communicates its requirements.

- Configuration Loss: Remember that this process is destructive to the EXOS configuration. Ensure you have a backup of your EXOS lab settings if you ever plan to revert.

Extreme Networks +2

Would you like the exact VOSS CLI syntax for your four specific VLANs and I-SID mappings?

9 sites

Extreme Networkshttps://documentation.extremenetworks.com

Migration to Segmented Management Instance - VOSS User Guide

Aug 15, 2022 — * Migrate a VLAN or CLIP IP address to the Segmented Management Instance. * Configure a Segmented Management Instance Using quick-

Extreme Networkshttps://documentation.extremenetworks.com

Procedure to Switch from VOSS to EXOS

Change the Network Operating System Personality. Use this procedure on a VOSS switch to change the Network Operating System (NOS) ...

Extreme Portalhttps://extreme-networks.my.site.com

Switch Operating System Selection for Universal Hardware

Jun 12, 2025 — To change the switch OS via the Bootloader menu, complete the following steps: * Connect the switch to a console with a baud rate ...

Show all

Yes but first, I want to backup all the EXOS configuration in my switches, and APs including the policies and all the CLI commands that I had to set on the 5320s.

Before you convert your lab to VOSS, you should capture a full record of your current setup. Because the VOSS migration wipes the local storage of the switch, relying on local backups is not enough.

1. Backup Extreme 5320 Switches (EXOS)

You should save both the binary database file and a human-readable CLI script.

- Human-Readable Script (.xsf): This is best for seeing exactly what commands you used.

  - Command: upload configuration <TFTP_SERVER_IP> my_lab_backup.xsf vr VR-Default.

  - Alternative: Run show configuration and copy-paste the output into a text file.

- Configuration Database (.cfg): This is the official backup file used for restoration.

  - Command: tftp put <TFTP_SERVER_IP> primary.cfg vr VR-Default.

- In XIQ: If your switches are "CLI-mode" managed, go to Configure > Devices > Switches, select your switch, and use Backups > Create Backup.

Extreme Portal +4

2. Backup AP3000 Access Points

Since APs are cloud-managed, their configurations are stored in your Network Policy.

- Export Policy: In ExtremeCloud IQ, go to Configure > Network Policies. You can view or clone the policy used for your

AP3000s

.

- Manual CLI Record: There is no "export file" for APs in XIQ. To see the specific CLI commands pushed to an AP:

  - Go to Manage > Devices.

  - Click the Configuration Audit (green icon) next to the AP.

  - Go to the Complete tab and copy the CLI text to a file.

Extreme Portal

Sequence Table: EXOS to VOSS Migration

| Step | Goal | EXOS / AP Action | VOSS (Fabric Engine) Action |
| --- | --- | --- | --- |
| 1 | Preservation | save then upload configuration to TFTP/USB. | N/A |
| 2 | OS Conversion | Select VOSS in XIQ ("Change OS") or use download image in CLI. | Switch reboots and formats flash for VOSS. |
| 3 | Management IP | Configure in-band/OOB IP to regain access. | interface vlan <ID> -> ip address <IP>. |
| 4 | The Backbone | N/A (Traditional Trunks/LAGs). | Enable IS-IS on core ports to build the Fabric. |
| 5 | VLAN Services | Create VLANs 1-4. | Create VLANs and map each to a unique I-SID. |
| 6 | AP Handshake | Configure Tagged ports for APs. | Enable Fabric Attach (FA) on AP ports for auto-provisioning. |

Summary of Differences

- Logic: EXOS is "VLAN-centric" (you must define paths everywhere). VOSS is "Service-centric" (you define the service at the edge, and the Fabric finds the path).

- Automation: In VOSS, once Auto-Sense is enabled, the switch detects the

AP3000

and automatically creates the required VLANs based on the AP's requests.

Extreme Networks Community

Would you like the exact CLI commands for mapping your 4 VLANs to I-SIDs in VOSS now?

9 sites

Extreme Portalhttps://extreme-networks.my.site.com

How To: How to Backup and Restore a Switch Engine (EXOS ...

Mar 31, 2026 — Procedure. There are two possible methods for this. The first method involves generating a XOS Scripting File (. xsf) from the con...

AnalysisManhttps://www.analysisman.com

Extreme Switch - How to backup/restore configuration in EXOS

Feb 3, 2018 — ▶ Configuration Backup * 1. Use the show switch command to see what the currently selected configuration file is. X460G2-24p-G4.8 ...

Extreme Networks Communityhttps://community.extremenetworks.com

Migrate exos Onprime to XIQ cloud - Extreme Networks - 116529

Nov 6, 2024 — ‎11-06-2024 04:57 AM. Hi, As I said - XIQ manages only configuration maintained by Switch Template and Network Policy settings. So...

Show all

Yes but before that outline the major architectural and service enabled differences for EXOS and VOSS side by side. Then give me the VOSS - CLI commands pls in a table with EXOS on the left and VOSS on the right for each 'service or module' deployment or configuration.

To move from EXOS to VOSS, you are shifting from Hop-by-Hop Bridging (where you manually touch every switch) to Fabric Networking (where the core is automated and you only configure the edges).

Architectural Differences

| Feature | EXOS (Switch Engine) | VOSS (Fabric Engine) |
| --- | --- | --- |
| Control Plane | STP (Spanning Tree) / OSPF / BGP | SPB (Shortest Path Bridging) / IS-IS |
| Traffic Path | Pre-defined "Tree" (Blocks redundant links) | Shortest Path (Uses all links, no loops) |
| VLAN Extension | Manual Tagging (802.1Q) on every trunk | I-SID Mapping (Configure at edge only) |
| Redundancy | MLAG (Requires peer-link) | vIST (Logical peer-link, no physical block) |
| AP Connectivity | Manual VLAN tagging on ports | Fabric Attach (FA) (Auto-provisioning) |
| Device Detection | LLDP (Informational) | Auto-Sense (Dynamic configuration) |

Configuration Migration: EXOS vs. VOSS CLI

Here is how your specific home-lab services translate from EXOS commands to VOSS commands.

| Service / Module | EXOS Command (Current) | VOSS CLI (New) |
| --- | --- | --- |
| 1. Enable Fabric / NNI | create sharing 1:1...(LAG) | int gig <port> isis enable isis spbm 1 |
| 2. Management VLAN | create vlan "Mgmt" conf vlan Mgmt ip 10.1.1.2/24 | vlan create 100 name "Mgmt" type port-mstprstp 0 interface vlan 100 ip address 10.1.1.2/24 |
| 3. User VLANs | create vlan "Data" conf vlan Data tag 10 | vlan create 10 name "Data" type port-mstprstp 0 vlan i-sid 10 10010 (Maps to Fabric) |
| 4. AP Port Config | conf vlan Data add port 5 tagged | int gig <port> fa enable auto-sense enable |
| 5. IP Routing | conf vlan Data ipaddress 10.10.1.1/24 enable ipforwarding vlan Data | interface vlan 10 ip address 10.10.1.1/24 (Routing is implicit in VOSS) |
| 6. Global Fabric ID | N/A | spbm spbm ethertype 0x8100 isis system-id <Unique_ID> |

Practical VOSS Deployment Steps for your 4 VLANs

Once you have converted the OS, enter config terminal and run these for your VLANs:

- Define the Services:

voss

vlan create 10,20,30,40 type port-mstprstp 0

vlan i-sid 10 10010

vlan i-sid 20 10020

vlan i-sid 30 10030

vlan i-sid 40 10040

Use code with caution.

- Activate the AP Ports:

voss

interface GigabitEthernet 1/1,1/2  # Use your actual AP ports

auto-sense enable

fa enable

exit

Use code with caution.

Pro Tip: In VOSS, once you configure the NNI (Network-to-Network Interface)between your two 5320s, the I-SIDs (VLANs) will automatically "stretch" between the switches without you having to tag them on the uplink ports.

Would you like the specific Global SPBM setup commands to ensure your two switches talk to each other correctly?

Yes pls, but before that, highlight the RFCs or IEEE standards which define how Fabric is architected, designed, configured, deployed, optimized.

The architecture of VOSS (Fabric Engine) is rooted in the Shortest Path Bridging (SPB)standard, which replaces traditional Spanning Tree (STP) with a more robust link-state routing approach.

IEEE Region 5 +2

Core Standards for Fabric Connect

- IEEE 802.1aq (Shortest Path Bridging): The primary standard defining the architecture of the Fabric. It enables a loop-free, multi-path Ethernet mesh using the shortest possible paths for all traffic.

- RFC 6329 (IS-IS Extensions for SPB): Defines the control plane. It uses the IS-IS link-state protocol to build the network topology and distribute service information (I-SIDs) without needing an IP stack.

- IEEE 802.1ah (Provider Backbone Bridging): The data plane standard. It uses MAC-in-MAC encapsulation to separate customer traffic from the backbone and hide user MAC addresses from the core switches.

- IEEE 802.1Qcj (Fabric Attach): Currently the basis for Fabric Attach (FA). It automates the connection between non-fabric devices (like your AP3000) and the Fabric, allowing them to dynamically request and join services.

- IEEE 802.1ag (Connectivity Fault Management): Defines the OAM (Operations, Administration, and Management) standards used for troubleshooting and optimizing the fabric paths.

IETF | Internet Engineering Task Force +10

Global SPBM and Configuration Migration

This table outlines the essential setup for your two-switch 5320 lab.

| Service / Module | EXOS CLI (Current) | VOSS CLI (New / Fabric) |
| --- | --- | --- |
| Global Fabric ID | N/A (Standard STP) | spbm spbm ethertype 0x8100 isis system-id <Unique_ID> |
| Backbone Links (NNI) | create sharing 1:1...(LAG) | interface gigabitethernet <port> isis enable isis spbm 1 |
| Management VLAN | create vlan "Mgmt" conf vlan Mgmt ip 10.1.1.2/24 | vlan create 100 name "Mgmt" type port-mstprstp 0 interface vlan 100 ip address 10.1.1.2/24 |
| User VLANs (1-4) | create vlan "Data" conf vlan Data tag 10 | vlan create 10,20,30,40 type port-mstprstp 0 vlan i-sid 10 10010 |
| AP Connectivity | conf vlan 10 add port 5 tagged | interface gigabitethernet <AP_Port> fa enable auto-sense enable |
| Routing Services | enable ipforwarding vlan 10 | interface vlan 10 ip address 10.10.1.1/24 ip ospf enable (if needed) |

Global SPBM Setup (Required First)

Before the links will form, you must define the Fabric parameters on each switch:

voss

# On Switch 1

router isis

system-id 0000.0000.0001

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.01

exit

Use code with caution.

(On Switch 2, use system-id 0000.0000.0002 and nick-name 0.00.02.)

Once these are set and you enable isis on the physical uplink ports, your switches will automatically form a fabric adjacency.

Would you like to review the Verification commands to confirm your two 5320s have successfully joined the SPB fabric?

17 sites

IETF | Internet Engineering Task Forcehttps://datatracker.ietf.org

RFC 6329 - IS-IS Extensions Supporting IEEE 802.1aq ...

Standards Track [Page 3] RFC 6329 IS-IS Support of IEEE 802.1aq April 2012 1. Introduction 802.1aq Shortest Path Bridging (SPB) [8...

IEEE 802.1https://1.ieee802.org

802.1aq – Shortest Path Bridging |

802.1aq – Shortest Path Bridging. ... The full title of this PAR is “Standard for Local and Metropolitan Area Networks: Virtual Br...

IEEE Region 5https://r5.ieee.org

Shortest Path Bridging IEEE Standard 802.1aq

Ms. Generra Oliver. Mr. Marquise Cooper. Page 2. IEEE Standard 802.1aq (SPB) Defined. • Shortest Path Bridging is a protocol inten...

Show all

Yes, pls, but before that can you provide an E2E architecture and the comparative data/ control flow between an EXOS and VOSS deployment, taking for example, one switch, with one AP that is connected via an XIQ? Or is this too simple?

It’s not too simple at all—it’s the best way to see how the "brain" of the network changes. While both use XIQ for management, the way the switch handles the AP's traffic is fundamentally different.

E2E Architecture: Switch + AP3000 + XIQ

In both scenarios, XIQ acts as the Management Plane (sending configuration via HTTPS/SSH). The difference lies in how the Control Plane and Data Plane handle the VLANs.

Comparative Flow: EXOS vs. VOSS

| Component | EXOS (Traditional Bridge) | VOSS (Fabric Connect) |
| --- | --- | --- |
| Control Plane | LLDP / STP: Switch and AP exchange LLDP to see each other. STP blocks loops. | Fabric Attach (FA): The AP and Switch use LLDP + FA extensions to "handshake" and negotiate services. |
| VLAN Provisioning | Static/Manual: You must manually create VLAN 10 on the switch port and tag it (conf vlan 10 add port 1 tagged). | Dynamic (Auto-Sense): You enable auto-sense. When the AP asks for "VLAN 10," the switch creates it and maps it to an I-SID on the fly. |
| Data Flow (Inner) | 802.1Q: Traffic leaves the AP with a standard VLAN tag. The switch looks at the MAC table and forwards. | MAC-in-MAC (802.1ah): Traffic enters the switch, gets encapsulated in a Fabric header, and is sent across the mesh via the shortest path. |
| The "Uplink" | Trunk/LAG: You must manually tag every VLAN (10, 20, 30, 40) on the link to the next switch. | NNI (IS-IS): You only configure the link as "Fabric." All I-SIDs (VLANs) flow across it automatically without manual tagging. |

End-to-End Traffic Path Example (User on Wi-Fi)

- User Connects: A phone joins the "Guest" SSID (VLAN 20) on the AP3000.

- AP to Switch:

  - EXOS: The AP sends the packet with a VLAN 20 tag. The switch port must already be configured to accept Tag 20.

  - VOSS: The AP tells the switch via Fabric Attach: "I need a path for VLAN 20." The switch dynamically opens that path.

- Switch to Core/Gateway:

  - EXOS: The packet stays a standard Ethernet frame. Every switch in the path must have "VLAN 20" manually created and tagged on its ports.

  - VOSS: The switch wraps the packet in a Fabric Header. The rest of the network doesn't need to know what "VLAN 20" is; they just route the Fabric Header to the destination Nick-Name.

The XIQ Factor

In EXOS, XIQ pushes a "Port Template" that defines specific VLANs. In VOSS, XIQ pushes an "Auto-Sense" policy, which is much more flexible because the switch "learns" what the AP needs rather than being told exactly what to do.

Would you like to see the VOSS verification commands to confirm the Fabric Attach handshake is working between your AP and the 5320?

Yes pls, but first let me get this straight. Unlike in EXOS where I had to manually push the switch-policy on each switch in the network to provision for the VLANs etc. in the VOSS case, I only need to define the 'intent' on the edge-switches and they automatically find the 'path' to the destination with me setting anything else up? If that is true, then we only need to configure the edge switch and something like a c core but where are the policies for these in XIQ related for the Fabric and pushed?

Your assessment is exactly correct. You are moving from a Hop-by-Hop model (EXOS) where you define the path to a Service-at-the-Edge model (VOSS) where the Fabric automates the core.

1. The "Intent-Based" Difference

- EXOS (Manual Pathing): To extend a VLAN from a switch to the core, you must manually create and tag that VLAN on every single uplink in the path.

- VOSS (Fabric Connect): You define the Service (VLAN/I-SID) only at the edge where the device (like your AP3000) connects. The Fabric—using Shortest Path Bridging (SPB)—automatically finds the fastest path between those I-SIDs across your backbone. You do not need to touch the uplink or core configurations for new VLANs.

Reddit +4

2. Where are policies pushed in XIQ?

In ExtremeCloud IQ (XIQ), the management of Fabric Engine (VOSS) is handled through Network Policies, but the type of policy changes:

- Fabric Attach (FA) Policy: Instead of "Port Templates" that just tag VLANs, you create Fabric Attach mappings. You link a VLAN ID to an I-SID within the policy.

- Auto-Sense Policy: You apply a "Universal" port type to your access ports. When an AP3000 is plugged in, the switch uses Auto-Sense to detect it and dynamically applies the FA policy.

- Configuration Location:

  - Go to Configure > Network Policies.

  - Select your policy and go to the Switching > Device Template section.

  - Look for Fabric Attach settings to define your VLAN/I-SID mappings.

Extreme Networks +4

3. Comparison of Policy Data Flow

| Component | EXOS Flow (Policy-to-Switch) | VOSS Flow (Policy-to-Switch) |
| --- | --- | --- |
| Backbone | XIQ pushes static "Trunk" configs to all uplinks. | XIQ pushes global IS-IS/SPBM settings; uplinks are self-forming. |
| Edge Ports | Port is statically set to "Tagged VLAN 10". | Port is set to Auto-Sense. |
| Device Onboarding | Switch waits for a device; VLAN 10 is always "live". | Fabric Attach handshake occurs; the switch only "activates" the I-SID when the AP asks for it. |
| Updates | If you add a VLAN, you must update policies for all switches in the path. | If you add a VLAN, you update the policy for the edge switch only. |

Summary of Setup

You only need to configure the Global Fabric parameters once (System ID, Nickname). After that, the Core switch stays "quiet"—you never have to add another VLAN to it manually again.

Would you like the Verification commands to see if your AP3000 has successfully "requested" its services from the 5320?

12 sites

Extreme Networkshttps://documentation.extremenetworks.com

Fabric Engine v9.3 User Guide - Product Documentation

Sep 12, 2025 — Enable LLDP Authentication of IP Phones. Configure Auto-sense Voice Information for IP Phones. Disable Auto-sense DHCP Server Dete...

Reddithttps://www.reddit.com

VOSS Information : r/ExtremeNetworks - Reddit

Feb 7, 2024 — You assign ISID/NSIs to the vlans on both Exos and Voss. Then you tell Voss the ports that are connected to Exos are Fabric Attach...

YouTube·Extreme Networkshttps://www.youtube.com

Extreme Fabric Connect Automation and Extension to the Edge

Jun 18, 2021 — and he will be showing you um the technical details on how this really works in action so I'm going to talk about it in slides. an...

3m

Show all

Yes but first a few items. I have two VLANs on each of the 5320 to carry traffic on two SSIDs per AP3000 that are connected. Are you saying that I only need to define the service-options say, alpha-bravo-delta-gamma on the edge switches somehow corresponding to the four VLANs for the UE traffic hanging on the APs and the Fabric does the rest?

That is exactly correct. In the VOSS world, you have just described Service-at-the-Edge.

In your EXOS setup, you had to build a "pipe" (VLAN tagging) through every switch link. In VOSS, you define the Service (I-SID) at the entry point, and the Fabric treats the core as a "black box" that automatically transports that service to any other point where that I-SID is defined.

How it works for your 4 VLANs:

- Define Once: You create your four VLANs (Alpha, Bravo, Delta, Gamma) on the switches and map them to unique I-SIDs (e.g., 1001, 1002, 1003, 1004).

- The "Cloud" Effect: Once those I-SIDs are mapped, the two 5320s "advertise" to each other via IS-IS: "I have service 1001 available here."

- Automatic Pathing: When a user on an AP connected to Switch A wants to reach a resource on Switch B, the Fabric automatically encapsulates that traffic and sends it across the NNI (uplink). You never have to manually tag those 4 VLANs on the link between the switches.

The AP3000 "Magic" (Fabric Attach)

Because you have AP3000s, it gets even simpler. You don't even have to manually "port map" the VLANs to the AP ports.

- The AP3000 speaks Fabric Attach (FA).

- When the AP boots, it tells the switch: "I am an AP, and I need I-SIDs 1001 and 1002 for my SSIDs."

- The 5320 says: "Copy that," and automatically provisions those VLANs on that specific port.

Comparison: Mapping Services

| Feature | EXOS Manual Method | VOSS Fabric Method |
| --- | --- | --- |
| Uplink Config | conf vlan Alpha add port 49 tagged (Repeat for all 4 VLANs) | Zero config. (IS-IS handles it globally). |
| Edge Port | conf vlan Alpha add port 1 tagged | Auto-Sense. (Port detects the AP and maps the VLAN). |
| Provisioning | Static: VLANs are always "on" the wire. | Dynamic: Service is only active when a device requests it. |

The VOSS CLI Verification Commands

Now that the architecture is clear, here is how you check if your "Intent" is working:

- Check the Fabric Backbone (The "Path"):
show isis adj
Look for "L1-2" status. This confirms your two 5320s are talking.

- Check the Services (The "Intent"):
show i-sid
Confirms your 4 VLANs are mapped to I-SIDs.

- Check the AP Connection (Fabric Attach):
show fa assignment
This is the "Gold" command. It shows you exactly which VLANs the AP3000 has automatically requested and if the switch granted them.

Would you like to see a sample XIQ Fabric Attach hierarchy to see exactly where to input those Alpha-Bravo-Delta-Gamma names and I-SIDs?

Yes pls, but before that one item on my E2E architecture. I connect my AP to 5320 which connects to my home modem which then connects to the internet. On EXOS I had to configure my home-modem to have a static route back to the two switches plus configure static routes from the VLAN gateways to the home modem. What to do here in VOSS?

In VOSS (Fabric Engine), the routing logic changes because the Fabric automates the pathing between your switches, but the connection to your Home Modem (the "Exit Point") still requires a defined gateway.

The "Internet Exit" Strategy

Unlike EXOS where you often manage routing hop-by-hop, VOSS uses Inter-VSN Routing. You will still need a default route on your switches to find the modem, and your modem still needs to know how to find your VLANs.

- Switch Default Route: You only need to configure the Default Static Route on the switch that is physically connected to your modem.

- Modem Static Routes: Your home modem still needs a static route for each of your 4 VLANs (Alpha, Bravo, etc.) pointing to the Switch's IP on the shared "Modem VLAN."

- Fabric Advantage: Once the switch connected to the modem knows the path to the internet, it can "advertise" this default route to the other 5320 automatically via the Fabric (using a feature called IP Shortcut), so you don't have to manually configure routes on the second switch.

Extreme Networks +1

Comparison: Routing to Your Modem

| Routing Task | EXOS (Switch Engine) Action | VOSS (Fabric Engine) Action |
| --- | --- | --- |
| Def. Gateway | configure iproute add default <Modem_IP> | ip route 0.0.0.0 0.0.0.0 <Modem_IP> weight 1 enable |
| Inter-VLAN | enable ipforwarding vlan <Name> | Implicit; once an IP is on the VLAN interface, it routes. |
| Fabric Core | Manual static routes on every switch. | IP Shortcut: Core switches learn the route via IS-IS. |
| Return Path | Modem needs routes for every VLAN. | Modem still needs routes for every VLAN. |

XIQ Fabric Attach Hierarchy

To provision your Alpha, Bravo, Delta, Gamma services in XIQ for your

AP3000

, follow this logical flow:

- Network Policy: Open your policy and navigate to Switching > Fabric Attach.

- Service Definition: Add a new entry for each SSID/VLAN.

  - VLAN ID: e.g., 10 (Alpha)

  - I-SID: e.g., 10010

- Port Profile: Assign a "Universal" or "Auto-Sense" profile to the 5320 ports where your APs connect.

- The Result: When the AP3000 boots, it sends an FA request. XIQ’s policy tells the switch: "If an AP asks for Alpha, give it I-SID 10010." The switch then creates the VLAN and bridges it to the Fabric dynamically.

VOSS Verification Commands

| Command | Purpose | Expected Result |
| --- | --- | --- |
| show isis adj | Check switch-to-switch link. | State should be UP (L1-2). |
| show fa assignment | Check AP3000 handshake. | Shows VLANs dynamically assigned to the AP port. |
| show ip route | Check Internet reachability. | Look for 0.0.0.0/0 pointing to your modem. |
| show i-sid | Check Fabric Services. | Lists your 4 VLANs and their active I-SIDs. |

Would you like the exact CLI blocks for your 4 VLANs to copy-paste into your 5320s?

9 sites

Extreme Networkshttps://documentation.extremenetworks.com

Inter-VSN routing - VOSS User Guide - Extreme Networks

Oct 15, 2023 — Configure Auto-sense IS-IS Level 1 Metric. Configure the Auto-sense Wait Interval Globally. Configure the Auto-sense Wait Interval...

Reddithttps://www.reddit.com

VOSS Management Route? : r/ExtremeNetworks - Reddit

Aug 29, 2025 — config. interface mgmtEthernet. ip address x.x.x.x/yy. default-gateway z.z.z.z. Afterwards, run show ip route vrf mgmt.

Extreme Portalhttps://extreme-networks.my.site.com

How to Create a Static Route on VOSS or Fabric Engine

Sep 19, 2025 — Procedure. The first and second specify the IP address and mask for the route destination. The third specifies the IP address of t...

Show all

Yes pls

Here are the exact CLI blocks to configure your two 5320 switches.

Note: Before pasting, ensure you have already performed the "Change OS" to VOSS via XIQ or the Bootloader. These commands assume Switch 1 is your "Gateway" switch (physically connected to your modem).

1. Global Fabric Setup (Do this first)

This builds the "engine" that allows the two switches to talk without manual tagging.

Switch 1 (Gateway Switch)

voss

conf t

router isis

system-id 0000.0000.0001

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.01

exit

isis enable

Use code with caution.

Switch 2 (Remote Switch)

voss

conf t

router isis

system-id 0000.0000.0002

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.02

exit

isis enable

Use code with caution.

2. Connect the Switches (NNI Link)

Identify the ports connecting the two switches (e.g., Port 48).

voss

# Run on both switches

interface GigabitEthernet 1/48

isis enable

isis spbm 1

no shutdown

exit

Use code with caution.

3. Define the Services (Alpha, Bravo, Delta, Gamma)

Map your VLANs to I-SIDs. Use the same IDs on both switches.

voss

# Run on both switches

vlan create 10 name "Alpha" type port-mstprstp 0

vlan create 20 name "Bravo" type port-mstprstp 0

vlan create 30 name "Delta" type port-mstprstp 0

vlan create 40 name "Gamma" type port-mstprstp 0

vlan i-sid 10 10010

vlan i-sid 20 10020

vlan i-sid 30 10030

vlan i-sid 40 10040

Use code with caution.

4. Enable AP Connectivity (Auto-Sense/Fabric Attach)

Assume your AP3000s are on Port 1 of each switch.

voss

# Run on both switches

interface GigabitEthernet 1/1

auto-sense enable

fa enable

no shutdown

exit

Use code with caution.

5. Internet Routing (Switch 1 only)

Configure the port connected to your home modem (e.g., Port 47) and set the default route.

voss

# On Switch 1 only

vlan create 100 name "Modem_Link" type port-mstprstp 0

vlan members add 100 1/47

interface vlan 100

ip address <Switch_IP_on_Modem_Subnet> <Mask>

exit

ip route 0.0.0.0 0.0.0.0 <Home_Modem_IP> weight 1 enable

# This allows Switch 2 to learn the route automatically via the Fabric

router isis

ip-shortcut

exit

Use code with caution.

How to verify it worked:

- show isis adj: Ensure state is UP.

- show fa assignment: You should see the AP3000 automatically mapping SSIDs to VLANs 10, 20, 30, or 40.

- ping <Home_Modem_IP>: Verify the Switch can see the internet.

Would you like the Troubleshooting commands in case the IS-IS adjacency doesn't come up immediately?

Yes pls

If your Fabric doesn't form immediately, it’s usually due to a mismatch in the Global Parameters or a physical link issue. Use these commands to pinpoint the snag.

1. Check the Physical "Engine" (IS-IS)

The most common issue is the switches not seeing each other as neighbors.

| Command | What to Look For |
| --- | --- |
| show isis adj | State should be UP. If it’s INIT or blank, check physical cables or the spbm ethertype. |
| show isis interface | Ensure your NNI port (e.g., 1/48) shows L1L2 and Enabled. |
| show spbm | Ensure Ethertype is 0x8100 on both switches. If one is 0x88a8, they won't talk. |

2. Check the Service "Intent" (I-SIDs)

If the switches are talking but your traffic isn't passing, check the service mapping.

| Command | What to Look For |
| --- | --- |
| show i-sid | Ensure the I-SIDs (10010, 10020, etc.) are listed and Type is ELAN. |
| show vlan i-sid | Verify your VLAN IDs are correctly mapped to the right I-SID. |
| show isis spbm i-sid | Crucial: This shows you if Switch 1 "sees" the I-SIDs being advertised by Switch 2. |

3. Check the AP Handshake (Fabric Attach)

If the AP3000 isn't getting its VLANs, the Fabric Attach handshake failed.

| Command | What to Look For |
| --- | --- |
| show fa neighbor | You should see the AP3000 listed on the port. If not, check fa enable and auto-sense enable. |
| show fa assignment | Shows the dynamic VLAN-to-I-SID mapping requested by the AP. Status should be active. |
| show lldp neighbor | Verify the switch sees the AP3000 via LLDP (the carrier for FA). |

4. Connectivity & Routing

| Command | What to Look For |
| --- | --- |
| show ip route | Look for the 0.0.0.0/0 route. On Switch 2, it should show a next-hop of Switch 1's Nick-Name. |
| ping <Modem_IP> | Tests the Gateway Switch to Modem link. |

Quick Fix Cheat Sheet

- No Adjacency? Check if you remembered isis enable globally AND on the interface.

- Can't Ping between switches? Ensure the manual-area (e.g., 00.0001) is identical on both.

- AP not working? Ensure the Management VLAN on the AP (usually VLAN 1) is also mapped to an I-SID if you aren't using the default.

Do you want to see how to save your configuration to the internal flash so it survives a reboot on VOSS?

Yes

In VOSS, the saving process is slightly different than the save command you used in EXOS. VOSS uses a "Primary Config" file approach.

To save your configuration so it survives a reboot, use the following command:

voss

save config

Use code with caution.

Important VOSS Persistence Tips:

- The Default File: By default, this saves everything to a file named primary.cfg on the internal flash.

- Automatic Backup: Unlike EXOS, VOSS does not automatically save when you make changes. If you configure a VLAN and the power cuts before you type save config, that VLAN is gone.

- Verify the Save: You can verify the file exists and see when it was last updated with:
show file list

- View Pending Changes: If you want to see what is currently running (but not yet saved), use:
show running-config

One Last Home-Lab "Gotcha"

In a home-lab environment, if you are testing different scenarios, you might want to create a manual backup point before making a big change:
copy primary.cfg my-working-lab.cfg

This way, if you break the Fabric logic, you can easily restore your "known good" state using:
boot config my-working-lab.cfg

Would you like the steps to verify that your configuration successfully synchronized with XIQ after the VOSS conversion?

Yes

To verify that your VOSS switches have successfully synchronized with ExtremeCloud IQ (XIQ), you should check both the cloud interface and the switch CLI.

1. Verification in the XIQ Dashboard

The most immediate way to verify the sync is through the ExtremeCloud IQ Manage Devices page:

Extreme Portal +1

- Status Icon: Once the VOSS switch is connected, its status icon in the device list will change from Red to Green. This can take up to 5 minutes after the persona change.

- Configuration Audit:

  - Green Check: Your local switch configuration perfectly matches the XIQ Network Policy.

  - Orange/Yellow Icon: There is a mismatch. This often happens if you made manual CLI changes that XIQ doesn't know about yet.

- Update Status: Check the "Update Status" column. It should say Succeeded. If it says "Device Update Failed," you may need to perform a "Complete" push to override local manual changes.

Extreme Portal +5

2. Verification via Switch CLI

If you are logged into the switch, you can verify the status of the cloud agent directly:

Extreme Portal +1

- Primary Command:

voss

show application iqagent status

Use code with caution.

- What to look for:

  - Connection Status: Should be Connected.

  - Agent Oper State: Should be connected.

  - Last Poll/Health Status: Should be SUCCESS.

  - Server Address: Should point to hac.extremecloudiq.com.

Extreme Portal +1

3. Resolving Sync Issues

If the switch is Red or Audit Failed:

- Manual Override: Go to Manage > Devices, select your switch, and click Update > Perform Delta Configuration Update. If that fails, select the option to "Resolve Local Device Configuration which is out of sync".

- Restart the Agent: If the switch shows as disconnected in the cloud, you can force a reconnect from the CLI:

voss

conf t

application

no iqagent enable

iqagent enable

Use code with caution.

Extreme Portal +2

Pro Tip: The first time you onboard a VOSS device, XIQ typically forces a Complete Configuration Push to ensure the initial baseline is synchronized.

Extreme Networks Community +1

Since your AP3000s are part of this lab, would you like to see how to verify the Fabric Attach status from the ExtremeCloud IQ dashboard?

Extreme Networks

9 sites

Extreme Portalhttps://extreme-networks.my.site.com

How to add a VOSS Switch to ExtremeCloud IQ - Extreme Portal

Nov 4, 2025 — If you have multiple VIQs mapped to your email address, you will see the below screen before landing on the main XIQ page. Click '

Extreme Networkshttps://documentation.extremenetworks.com

Display ExtremeCloud IQ Agent Status - VOSS User Guide

Sep 15, 2022 — About this task. Use this task to display IQ Agent status information. Procedure. To enter User EXEC mode, log on to the switch. D...

Extreme Portalhttps://extreme-networks.my.site.com

Solution: XIQ: "Device Update Failed" with a SwitchEngine/EXOS or ...

Jul 18, 2023 — Resolution * Manually revert any configuration changes that were not made by XIQ, then attempt another configuration push. * Reset...

Show all

Yes pls

In ExtremeCloud IQ (XIQ), you can verify the Fabric Attach (FA) handshake between your AP3000 and the 5320 without even touching the CLI. This confirms that the switch has dynamically provisioned the I-SIDs for your SSIDs.

1. Verification in the XIQ GUI

- Device Level: Go to Manage > Devices, click on your 5320 Switch, and navigate to the Fabric Attach tab (or check the "Ports" view).

- The "Active Assignments" Table: Look for a table showing your AP's port. It should list:

  - VLAN/I-SID: Your Alpha, Bravo, etc., mappings.

  - Status: Should show as Active or Auth.

- Client Level: If you see clients connected to the SSIDs but the switch port shows no manual VLAN configuration, Fabric Attach is working perfectly—the switch is "plumbing" the connection on demand.

2. Verification via Switch CLI (The "Truth" Command)

The most definitive way to see what the AP3000 has requested from the switch is:

voss

show fa assignment

Use code with caution.

What to look for in the output:

- Port: The port where your AP is plugged in.

- VLAN/I-SID: The specific IDs you mapped for Alpha, Bravo, etc.

- State: Should say Active.

- Type: Should say Dynamic (meaning the AP asked for it, and the switch granted it).

Final "Lab Checklist" for your VOSS Deployment

- OS Change: Both 5320s converted to VOSS and showing Green in XIQ.

- NNI Check: show isis adj shows your link is UP.

- Service Check: show i-sid shows your 4 VLANs are mapped.

- AP Check: show fa neighbor shows the AP3000 is detected.

- Internet Check: ping 8.8.8.8 from a device on one of the SSIDs.

Comparison Summary

In your old EXOS setup, if you added a 5th VLAN, you had to update every switch and every uplink. In your new VOSS lab, you only add the VLAN and I-SID to the Network Policy in XIQ and push it—the Fabric handles the rest of the pathing automatically.

Do you have your VLAN IDs and I-SIDs decided for your Alpha, Bravo, Delta, and Gamma networks, or would you like a standardized numbering scheme recommendation for a home lab?

Let's go with standardized numbering

For a clean, scalable home lab, it is best practice to align your VLAN IDs with your I-SIDs. This makes troubleshooting much easier because the numbers match across the entire Fabric.

Here is a recommended standardized numbering scheme for your Alpha, Bravo, Delta, and Gamma networks:

Recommended Numbering Scheme

| Service Name | VLAN ID | I-SID | Subnet Example | Purpose |
| --- | --- | --- | --- | --- |
| Management | 10 | 10010 | 10.0.10.0/24 | Switches & AP Management |
| Alpha | 100 | 100100 | 10.0.100.0/24 | Primary User / Trusted Wi-Fi |
| Bravo | 200 | 100200 | 10.0.200.0/24 | IoT / Smart Home |
| Delta | 300 | 100300 | 10.0.300.0/24 | Guest Wi-Fi |
| Gamma | 400 | 100400 | 10.0.400.0/24 | Lab / Testing |

Why use this scheme?

- Logical Mapping: Adding 100,000 to the VLAN ID to create the I-SID is a standard convention. If you see I-SID 100200 in a trace, you immediately know it belongs to VLAN 200.

- Isolation: By keeping your Management VLAN (10) separate from your user traffic (100-400), you ensure that a broadcast storm on a user SSID won't lock you out of your switch management.

VOSS CLI Implementation (Standardized)

Copy and paste this into both 5320s to apply the scheme:

voss

# Create VLANs

vlan create 10,100,200,300,400 type port-mstprstp 0

# Map to I-SIDs

vlan i-sid 10 10010

vlan i-sid 100 100100

vlan i-sid 200 100200

vlan i-sid 300 100300

vlan i-sid 400 100400

# Name them for clarity

vlan name 10 "MGMT"

vlan name 100 "Alpha"

vlan name 200 "Bravo"

vlan name 300 "Delta"

vlan name 400 "Gamma"

Use code with caution.

Next Steps:

- Apply these settings in XIQ under your Network Policy > Fabric Attach.

- Perform a Complete Push to your switches.

Shall we move on to configuring the IP Interfaces (Gateways) for these VLANs so your clients can actually route to the internet?

I currently have VLAN 10, 20,30, 50,60

Since you already have VLANs 10, 20, 30, 50, and 60 defined in your current EXOS setup, we will map them directly to a standardized I-SID scheme in VOSS to keep the transition seamless.

Following the best practice of adding 100,000 to the VLAN ID for the I-SID, here is your standardized configuration:

Updated Home-Lab Service Map

| Service Name | VLAN ID | I-SID | Recommended Subnet |
| --- | --- | --- | --- |
| Management | 10 | 100010 | 10.0.10.0/24 |
| Alpha | 20 | 100020 | 10.0.20.0/24 |
| Bravo | 30 | 100030 | 10.0.30.0/24 |
| Delta | 50 | 100050 | 10.0.50.0/24 |
| Gamma | 60 | 100060 | 10.0.60.0/24 |

VOSS CLI Implementation (For your existing VLANs)

Run these commands on both 5320 switches once they are in VOSS mode:

voss

# Create the VLANs

vlan create 10,20,30,50,60 type port-mstprstp 0

# Map to standardized I-SIDs

vlan i-sid 10 100010

vlan i-sid 20 100020

vlan i-sid 30 100030

vlan i-sid 50 100050

vlan i-sid 60 100060

# Add names for easier XIQ/CLI management

vlan name 10 "MGMT"

vlan name 20 "Alpha"

vlan name 30 "Bravo"

vlan name 50 "Delta"

vlan name 60 "Gamma"

Use code with caution.

Important Verification for EXOS Migration

In EXOS, you likely had these VLANs tagged on your inter-switch trunks. In VOSS, stop here. Once you have enabled isis on your uplink ports (as we did in the previous step), these VLANs are now "stretched" between your switches automatically. You do not need to add these VLANs to your uplink ports manually.

Would you like the IP Interface commands (Gateways) for these specific VLANs on your primary switch to enable internet routing for your clients?

Where id I enable isis?

To enable IS-IS (the control plane for your Fabric), you must enable it in two places: globally to start the process and individually on each physical port that connects your switches together (the NNI links).

Extreme Networks +3

1. Global Enablement

This "turns on" the IS-IS routing engine for the entire switch. It should be the final step after you've defined your System ID and Nickname.

Extreme Networks +1

voss

conf t

router isis enable

exit

Use code with caution.

2. Interface Enablement (The NNI Links)

You must explicitly tell the switch which physical ports are "Fabric" ports. If you have your two 5320s connected on Port 48, run this on both switches:

Extreme Networks +1

voss

interface GigabitEthernet 1/48

isis

isis enable

isis spbm 1

exit

Use code with caution.

Why two steps?

- Global: Prepares the switch to participate in a Fabric.

- Interface: Specifically designates a port as a Network-to-Network Interface (NNI). IS-IS will only try to form a "handshake" (adjacency) on ports where it is enabled.

Extreme Networks +2

Quick Verification

Once enabled in both places, you can verify the status immediately:

- show isis: Verifies the global process is running.

- show isis interface: Verifies IS-IS is active on your specific ports.

- show isis adj: The most important command; it should show your other 5320 as a neighbor with a state of UP.

Extreme Networks +3

Note: You do not enable IS-IS on the ports connected to your AP3000s or your home modem. Those ports use Fabric Attach or standard routing, not the Fabric core protocol.

Shall I show you how to check for any errors if the show isis adj command comes back empty?

9 sites

Extreme Networkshttps://documentation.extremenetworks.com

Configure Minimum SPBM and IS-IS Parameters

Aug 15, 2022 — interface {GigabitEthernet {slot/port[/sub-port][-slot/port[/sub-port]][,... ]}| mlt <1–512> } Configure an IS-IS interface on the...

Extreme Networkshttps://documentation.extremenetworks.com

Configuring optional IS-IS global parameters - VOSS User Guide

Sep 15, 2022 — Procedure. Enter IS-IS Router Configuration mode: enable. configure terminal. router isis. Configure optional IS-IS global paramet...

Extreme Networks Communityhttps://community.extremenetworks.com

Recommended Method of Connecting Single Switch to vIST pair of ...

Nov 4, 2022 — That all ! ISIS will handle the topology. If you want to priorise links to another, define the spbm l1-metric on both side of link...

Show all

Yes

If show isis adj is empty, your switches aren't forming a "neighbor" relationship. In VOSS, this is usually due to a mismatch in the "Backbone" settings.

1. Check for the "Three Pillars" Mismatch

For a Fabric to form, these three settings must be logically aligned on both 5320s.

- Ethertype Mismatch: The most common culprit.

  - Check: show spbm

  - Fix: Ensure both switches show 0x8100. If one is different, they will drop each other's packets.

- Manual Area Mismatch:

  - Check: show isis

  - Fix: Ensure manual-area (e.g., 00.0001) is identical on both switches.

- System ID Conflict:

  - Check: show isis

  - Fix: Ensure system-id is unique for each switch (e.g., ...0001 vs ...0002). If they are the same, the adjacency will flap or never stay UP.

2. Check the Interface State

If the global settings are correct, the port might be the problem.

- show isis interface:

  - Look for the OperState. It should be Up.

  - If it says Down, check if the physical port is shut down (no shutdown).

- show isis error:

  - This will tell you if the switch is receiving IS-IS packets but rejecting them (e.g., "Mismatched Area" or "Bad Password").

3. Check MTU (The "Silent Killer")

Fabric headers (MAC-in-MAC) add overhead. While 5320s usually handle this automatically on NNI ports, if you have another device or media converter in between, it might drop the larger frames.

- Command: show isis interface

- Fix: Ensure the MTU on your NNI port is at least 1522 (default is usually higher for Fabric).

4. The "Nuclear" Verification

If all else fails, run this to see if the switch is even trying to send Fabric hellos:

voss

debug isis adjacency

Use code with caution.

(Type no debug all to stop the scrolling text once you see the error message.)

Once your adjacency shows UP, would you like to configure the IP routing so your VLANs can reach your home modem?

Yes pls

To get your clients out to the internet, we need to set up the IP interfaces (gateways) for your VLANs and point a default route toward your home modem.

In VOSS, we use IP Shortcuts. This is a "set it once" feature: once the Gateway Switch (Switch 1) knows the route to the modem, the Fabric automatically tells the Remote Switch (Switch 2) how to get there.

1. Configure the "Modem Link" (Switch 1 only)

Assuming your home modem is on Port 47 and its IP is 192.168.1.1.

voss

# Create a transit VLAN for the modem

vlan create 100 name "Internet_Exit" type port-mstprstp 0

vlan members add 100 1/47

interface vlan 100

ip address 192.168.1.2 255.255.255.0

exit

# Set the static default route to the modem

ip route 0.0.0.0 0.0.0.0 192.168.1.1 weight 1 enable

Use code with caution.

2. Configure VLAN Gateways (Both Switches)

You can choose to have the gateway live on Switch 1, or use Anycast IP (RSMLT) to have the gateway live on both switches simultaneously for better performance. For a home lab, let's keep it simple and put the IPs on your Gateway Switch (Switch 1).

voss

# On Switch 1

interface vlan 10

ip address 10.0.10.1 255.255.255.0

exit

interface vlan 20

ip address 10.0.20.1 255.255.255.0

exit

interface vlan 30

ip address 10.0.30.1 255.255.255.0

exit

# Repeat for VLAN 50 and 60...

Use code with caution.

3. Enable "IP Shortcuts" (The Magic Step)

Run this on both switches. This allows Switch 2 to "see" Switch 1’s routing table across the Fabric without needing its own direct connection to the modem.

voss

router isis

ip-shortcut

exit

Use code with caution.

4. Verification

Once applied, test the flow from the CLI of Switch 2:

- ping 192.168.1.1: Can Switch 2 see the modem across the Fabric?

- show ip route: You should see 0.0.0.0/0 with a "Next Hop" showing the Nick-Name of Switch 1.

Crucial Reminder: Your Home Modem still needs to know how to get back to your lab. You must log into your modem's web interface and add a Static Route: 10.0.0.0 / 255.255.0.0 (covering all your VLANs) pointing to 192.168.1.2 (the switch).

Does your home modem support adding static routes, or should we look at DHCP Relayso your devices can get IP addresses automatically?

Hold on, right now, I have the two EXOS switches separately connected to my home modem independently connecting to the internet and XIQ

That changes the architecture significantly! In your current EXOS setup, you have two independent "islands" both reaching out to the internet. If we move to VOSS (Fabric Connect), we can actually make this much more resilient and "Fabric-aware."

Since both switches have a physical path to the modem, you have two choices for your VOSS migration:

Option A: The "Fabric Core" Model (Recommended)

You keep the Fabric link (NNI) between the switches. One switch acts as the Primary Internet Exit, but if that switch fails, the Fabric automatically reroutes all traffic through the other switch to the modem.

- Benefit: Your VLANs (Alpha, Bravo, etc.) are "stretched." A guest on an AP at Switch 2 can talk to a device on Switch 1 without leaving the Fabric.

- Change: You only need to define the VLAN gateways once.

Option B: The "Dual-Homed" Model

You keep them completely independent (no NNI link between them).

- Reality: This isn't really a "Fabric"; it's just two standalone VOSS switches. You lose the ability to roam seamlessly between APs or share I-SIDs across the switches.

How to adapt the VOSS config for your "Independent" connections:

If you want to keep both connected to the modem while running a Fabric between them, do this:

1. Keep the NNI (The link between switches)
Keep that Port 48 connection with isis enable. This creates the "East-West" path for your VLANs.

2. Dual Internet Exits (North-South)
Since both are plugged into the modem, you configure the "Modem VLAN" (e.g., VLAN 100) on both switches.

voss

# On Switch 1

interface vlan 100

ip address 192.168.1.2 255.255.255.0

exit

# On Switch 2

interface vlan 100

ip address 192.168.1.3 255.255.255.0

exit

Use code with caution.

3. The Default Route
Each switch points to the modem locally.

voss

# On both switches

ip route 0.0.0.0 0.0.0.0 192.168.1.1 weight 1 enable

Use code with caution.

4. The "Fabric" Advantage (Anycast GW)
Instead of picking one switch to be the gateway for your users, use Anycast IP. You give VLAN 20 the exact same IP (10.0.20.1) on both switches.

- Result: If a user is on Switch 1, they exit via Switch 1's modem link. If they move to Switch 2, they exit via Switch 2's link—but their IP gateway never changes!

Comparison of Connectivity

| Feature | Your Current EXOS Setup | New VOSS Fabric Setup |
| --- | --- | --- |
| Inter-Switch | Independent (or manual trunking) | Self-healing Fabric (IS-IS) |
| VLAN Gateways | Independent per switch | Anycast (Shared IP across both) |
| Internet Path | Fixed per switch | Redundant: Switch 1 can use Switch 2's modem link if its own fails. |
| XIQ Management | Two separate device configs | One Unified Network Policy |

Does your home modem have enough ports to keep both switches plugged in, or would you prefer to use the Fabric to "backhaul" all internet traffic to just one main connection?

Option A pls. I'd like to see how the data migrates between the switches without having to go to the internet because the two APs on both the switches are carrying say, voice traffic?

This is where Fabric Connect (SPB) shines. In your old EXOS setup, voice traffic moving between switches often had to be "hairpinned" through a router or tagged across a manual trunk. In VOSS, that traffic stays entirely within the Fabric (Layer 2) and takes the direct "East-West" path.

The Voice Flow: Switch A to Switch B

Because you are using Option A, your two switches are linked via the NNI (IS-IS)port.

- The Handshake: Switch 1 and Switch 2 see each other via IS-IS. They both say, "I have I-SID 100020 (Alpha/Voice) active here."

- The Direct Path: When a VoIP phone on Switch 1 talks to a phone on Switch 2, the traffic enters the switch, gets wrapped in a MAC-in-MAC (802.1ah) header, and is sent directly across the physical link to the other switch.

- No Internet Needed: This happens purely at the hardware level. Even if your home modem is unplugged, the two APs (and their voice clients) will continue to talk to each other perfectly.

Configuration for "Anycast" Voice (Option A)

To make this work seamlessly so that a phone can move from AP 1 to AP 2 without dropping a call, we use Anycast Gateways.

Run this on BOTH switches:

voss

# 1. Ensure the Service exists

vlan create 20 name "Voice" type port-mstprstp 0

vlan i-sid 20 100020

# 2. Configure the Anycast Gateway

# Both switches share the same "Virtual" IP.

interface vlan 20

ip address 10.0.20.1 255.255.255.0

exit

# 3. Enable the Fabric Routing

router isis

ip-shortcut

exit

Use code with caution.

Why this is better for Voice:

- Zero-Delay Roaming: Since both switches have the IP 10.0.20.1, the phone never has to "re-discover" its gateway when the user walks from one AP to the other.

- Micro-Segmentation: The voice traffic (I-SID 100020) is logically invisible to your Guest or IoT VLANs. They are completely "dark" to each other unless you specifically write a policy to let them talk.

- Path Optimization: VOSS always calculates the shortest path. If you eventually add a third switch, the Fabric will automatically find the fastest way to get that voice packet to its destination.

Verification of East-West Traffic

To prove the switches are talking directly, run this on Switch 1:
show isis spbm i-sid
Look for I-SID 100020. You should see the Nick-Name of Switch 2 listed as a destination. This confirms the Fabric has built a hardware bridge between them.

Would you like to see how to prioritize this Voice traffic using QoS (Quality of Service) settings within the Fabric?

Yes pls

In VOSS (Fabric Engine), Quality of Service (QoS) is significantly easier to manage than in EXOS because the Fabric automatically preserves the priority of your traffic as it travels across the network.

1. The Fabric QoS Philosophy

- Edge Marking: You identify the voice traffic at the edge (where the AP connects).

- Fabric Transparency: Once the packet is inside the Fabric (MAC-in-MAC), the switch copies the priority bit from the inner packet to the outer Fabric header.

- No Core Config: You don't have to configure QoS on the links between switches; they automatically respect the "High Priority" tag on the Fabric header.

2. Implementation: Prioritizing Voice (VLAN 20)

We want to ensure that traffic on VLAN 20 (your Voice/Alpha network) is treated as "Critical" or "Voice" (typically 802.1p bit 5 or 6).

Run this on both switches:

voss

# 1. Map the VLAN to a high-priority CoS (Class of Service)

# This tells the switch: "Everything on VLAN 20 is Priority 6"

vlan config 20 default-control-priority 6

# 2. Configure the AP Port to trust the incoming tags from the AP3000

interface GigabitEthernet 1/1

qos 802.1p-override disable

exit

Use code with caution.

3. Traffic Flow with QoS

| Step | Action | Priority Level |
| --- | --- | --- |
| At AP3000 | AP tags a voice packet as "Voice". | CoS 6 |
| At Switch 1 | Switch receives packet on Port 1, sees CoS 6. | CoS 6 |
| In Fabric | Switch wraps it in a Fabric Header and copies CoS 6 to the outside. | CoS 6 (Protected) |
| At Switch 2 | Switch 2 sees the high priority, moves it to the "Express" queue. | CoS 6 |
| To AP 2 | Packet is delivered to the destination AP with priority intact. | CoS 6 |

4. Verification of QoS

To see if your traffic is actually hitting the high-priority queues, use these commands:

| Command | Purpose |
| --- | --- |
| show qos cos-status | Shows how many packets are hitting each queue (Look for Queue 6). |
| show interface vlan qos 20 | Verifies the priority setting for your Voice VLAN. |
| show port vlan-priority | Shows if any ports are overriding the priority settings. |

Summary of Benefit

In your old EXOS setup, you would have had to create a "QoS Profile" and apply it to every single trunk port in the path. In VOSS, by setting the priority at the VLAN level (vlan config 20...), the entire Fabric respects that priority automatically.

Now that your Voice traffic is prioritized and your Anycast Gateway is set, would you like to see how to test the failover—simulating what happens if the link between your switches is cut?

Yes pls

To test the failover in a VOSS Fabric, you are essentially testing Sub-Second Convergence. Unlike EXOS, which relies on Spanning Tree (STP) to "unblock" a port (taking 3–30 seconds), VOSS uses IS-IS to simply calculate a new path instantly.

The Failover Test Scenario

Since you have Option A (both switches connected to the modem and each other), we want to see what happens to a Voice call if the link between Switch 1 and Switch 2 is pulled.

Step 1: Start a Continuous "Service" Test

From a laptop or device on Switch 2 (VLAN 20), start a continuous ping to the Gateway IP or an internet address:

- ping 10.0.20.1 -t (Anycast Gateway)

- ping 8.8.8.8 -t (External)

Step 2: Simulate the Failure

Physically unplug the NNI cable (e.g., Port 48) that connects the two 5320s.

Step 3: Observe the Convergence

- EXOS Result (Old): You would likely see 3–10 dropped pings while STP or MLAG re-converged.

- VOSS Result (New): You should see zero to one dropped ping. The Fabric realizes the "East-West" path is gone and immediately switches that VLAN's traffic to use the "North-South" link (the direct connection to the modem) for that switch.

Verification Commands during Failure

Run these while the cable is unplugged to see the Fabric "self-heal":

| Command | Expected "Fail" State Result |
| --- | --- |
| show isis adj | The adjacency to the other switch will be gone. |
| show ip route | The 0.0.0.0/0 route will now point strictly to the local modem interface rather than the other switch's Nick-Name. |
| show isis spbm i-sid | You will see that the I-SIDs (20, 30, etc.) no longer have a "Remote" destination; they are now "Local Only" on that switch. |

Step 4: Recovery (The "Auto-Heal")

Plug the cable back in. VOSS will detect the neighbor, re-establish the IS-IS adjacency, and move the traffic back to the optimal path without any manual intervention or "Preempt" settings.

Why this works for Voice

Because you configured Anycast IP (10.0.20.1 on both switches), the phone doesn't care which switch it talks to. Even if the link between switches dies, the AP3000 on Switch 2 just sends the voice data to its local copy of the gateway on Switch 2, and it goes straight to the modem.

Would you like to see how to monitor these "link down" events in the XIQ Event Logs to see the exact millisecond convergence time?

Yes pls

To monitor failover events and convergence times in ExtremeCloud IQ (XIQ), you use the Event Logs and Device Alarms. Because VOSS uses IS-IS, the logs will show exactly when the "Adjacency" went down and when the services rerouted.

1. Locating the Logs in XIQ

- Go to Manage > Events.

- Click the Filters (funnel icon) and select your two 5320 switches.

- Look for Category: System and Severity: Critical/Major.

2. What to look for in the Event List

When you pull the NNI (Fabric) cable, you will see a sequence like this:

- IS-IS Adjacency Down: "Neighbor 0.00.02 on port 1/48 changed state to DOWN."

- L3 Gateway Transition: If you are using Anycast, you may see an event stating the path to the gateway has changed.

- Fabric Attach Updates: If the AP3000 has to re-signal its services, you will see an "FA Assignment Updated" log.

3. Checking Millisecond Convergence (CLI)

XIQ events are great for history, but for exact millisecond precision, the VOSS switch keeps a "local" record of how fast it recalculated the path. Run this command on the switch:

voss

show isis spbm

Use code with caution.

Look for the line: "Last SPF Duration".

- In a 5320 home lab, this should be under 50ms. This is why your voice call doesn't drop—the human ear cannot detect a gap that small.

4. Setting up Alerts

To get a "Push Notification" or Email when a failover happens:

- Go to Monitor > Alarms.

- Click Alarm Settings.

- Ensure "IS-IS Adjacency Change" and "Interface State Change" are checked.

- You can now receive an alert the moment your lab link fails.

Summary of the "Option A" Success

You have now moved from a manual EXOS environment to a Self-Healing Fabric.

- Voice is prioritized (QoS).

- Gateways are redundant (Anycast).

- Management is centralized (XIQ).

- Convergence is sub-second (SPB/IS-IS).

Would you like to explore ExtremeCloud IQ's "Network 360" view to see a visual map of how your Fabric and APs are currently linked?

Yes pls

In ExtremeCloud IQ (XIQ), the Network 360 view acts as your central dashboard for visualizing how your Fabric and APs are linked. It provides a real-time, color-coded health assessment of your entire network, from the wired Fabric core to individual wireless clients.

YouTube +2

Accessing the 360 View

- Navigate to ML Insights: Click on the ML Insights tab in the main navigation menu.

- Select Network 360 Monitor: This provides a bird's-eye view of your global locations, sites, and health scores.

- Drill Down: You can filter by specific sites to see Topology or Floor Plan maps, which are critical for seeing how your two 5320s and AP3000s are interacting.

extr-p-001.sitecorecontenthub.cloud +3

Visualizing the Fabric and APs

- Topology View: This map displays the logical links between your switches. For your Fabric Engine (VOSS) setup, you can specifically visualize IS-IS areas and Fabric links(often shown as purple links in related tools) to confirm your NNI connection is healthy.

- Floor Plan Maps: If you have uploaded a floor plan, you can place your AP3000s on the map to see real-time signal coverage and where clients are currently connected.

- Device Trails: You can even track a client's "roaming trail" as they move between your two APs, which is perfect for confirming that your sub-second voice failover is working in practice.

extr-p-001.sitecorecontenthub.cloud +5

Key Benefits for Your Lab

- Health Scorecard: XIQ assigns a health score to categories like Devices, Clients, and Wi-Fi, allowing you to spot issues before they impact your voice traffic.

- Interactive Diagnostics: You can right-click any device on the map to access detailed Device 360 metrics, including CPU usage, port status, and Fabric Attach (FA) handshake details.

YouTube +2

Would you like help setting up a Floor Plan in XIQ to precisely position your AP3000s for coverage testing?

9 sites

YouTube·Extreme Networkshttps://www.youtube.com

ExtremeCloud IQ - Public Cloud Walk-Through

Aug 24, 2020 — or even on premises. on your site behind your own firewalls. so let's get started. right now I'm in the network 360 monitor tab of...

1m

YouTube·Extreme Networkshttps://www.youtube.com

Visibility, Management, and Troubleshooting Using ...

Jun 18, 2021 — hi my name is Ken Jones and I'm from the technical marketing engineering team here at Extreme Networks. i'm here to talk to you to...

35s

extr-p-001.sitecorecontenthub.cloudhttps://extr-p-001.sitecorecontenthub.cloud

ExtremeCloud™ IQ Site Engine

Site Engine includes tools to create topology and geographic maps of devices and floor plans of wireless APs on a network. The Fab...

Show all

Not right now. I want to create the home-lab migration. I want an E2E HLD and LLD to do that. First with topology/ architecture.

To build your migration from EXOS to VOSS (Option A), here is the structured High-Level Design (HLD) and Low-Level Design (LLD). This ensures your "Voice" and "Data" traffic remains local and resilient.

1. High-Level Design (HLD)

Architecture: "The Resilient Fabric Edge"

- The Core: Two 5320 switches connected via a high-speed NNI (Network-to-Network Interface). This forms the "Fabric Connect" cloud using 802.1aq (SPB).

- The Services: Instead of VLAN trunks, we use I-SIDs (I-SID 100010–100060). These services are "stretched" across the switches automatically.

- The Edge: AP3000s connect via Fabric Attach (FA). They dynamically request the VLANs they need from the switch.

- The Exit: Both switches are physically connected to the Home Modem. If Switch 1's link fails, Switch 2 provides the path to the internet for the entire Fabric.

2. Low-Level Design (LLD)

A. Physical Topology

- Inter-Switch Link (NNI): Port 1/48 (Switch 1)

↔

↔

Port 1/48 (Switch 2).

- Modem Link: Port 1/47 on both switches

→

→

Home Modem.

- AP Links: Port 1/1 on both switches

→

→

AP3000.

B. Logical Service Map

| Service | VLAN | I-SID | IP Gateway (Anycast) | Priority (QoS) |
| --- | --- | --- | --- | --- |
| MGMT | 10 | 100010 | 10.0.10.1 | 6 |
| Alpha (Voice) | 20 | 100020 | 10.0.20.1 | 6 |
| Bravo | 30 | 100030 | 10.0.30.1 | 2 |
| Delta | 50 | 100050 | 10.0.50.1 | 1 |
| Gamma | 60 | 100060 | 10.0.60.1 | 1 |

C. Control Plane (IS-IS / SPBM)

- Ethertype: 0x8100

- Manual Area: 00.0001

- System IDs: 0000.0000.0001 (SW1) and 0000.0000.0002 (SW2).

- Nicknames: 0.00.01 (SW1) and 0.00.02 (SW2).

3. Step-by-Step Migration Sequence

- Backup: Capture EXOS show configuration and XIQ AP policies.

- Persona Change: In XIQ, select both 5320s

→

→

Actions

→

→

Change OS to VOSS.

- Base Config: Access via CLI or XIQ to set the System-ID and Nickname.

- Fabric Build: Enable isis on Port 1/48 to form the link between switches.

- Service Provisioning: Create VLANs 10–60 and map to I-SIDs.

- Edge Activation: Enable auto-sense and fa enable on AP ports (1/1).

- Routing: Apply Anycast IPs to VLAN interfaces and set the static default route to your modem (192.168.1.1).

- Sync: Perform a Complete Push from XIQ to align the cloud policy.

Would you like me to generate the full "ready-to-paste" CLI script for each switch based on this LLD?

Hold on, I have a 16port 5320-16P-2MXT-2X. Am I connecting these in daisy chain? APs should go on port-3 on each switch Modem on Port-1. Where does the NNI go? Also, I want to deploy in two different ways one with full CLI and the other with XIQ. Is that possible?

The

Extreme Networks 5320-16P-2MXT-2X

is a high-performance, fanless 16-port compact switch. For your home-lab migration, you can absolutely deploy it using either a full CLI-driven method or a cloud-managed XIQ approach.

Extreme Portal +4

Physical Topology (Option A: Resilient Fabric)

You are not connecting them in a standard daisy chain; you are building an SPB Fabricwhere the switches are peers.

- Modem (Port 1): Connect your home modem to Port 1 on both switches. This provides redundant internet paths.

- Access Points (Port 3): Connect your AP3000s to Port 3 on each switch. Port 3 provides PoE+ to power the APs.

- NNI (Fabric Link): Use the high-speed SFP+ ports (U1 or U2) or the Multi-Gigabit ports (17 or 18) for the NNI.

  - Recommendation: Use Port 17 or 18 (the 1/2.5/5/10G RJ45 ports) for the link between your two switches to keep the SFP+ slots free for future 10G fiber needs.

CDW +4

E2E Deployment Methods

Method 1: Full CLI Deployment

Best for users who want total control over the local configuration.

Reddit

- Persona Change: Log in to the EXOS CLI and type y when prompted: "Would you like to change the switch OS to VOSS?".

- Fabric Base: Define your unique System-ID and Nickname for each switch.

- NNI Setup: Enable isis on the inter-switch port (e.g., Port 17).

- Service Config: Use vlan create and vlan i-sid to map your 10, 20, 30, 50, and 60 VLANs.

- Edge Config: Run fa enable and auto-sense enable on Port 3.

Extreme Portal +3

Method 2: XIQ Cloud Deployment

Best for automated management and visualization.

Extreme Networks

- Onboard: Add your switches to ExtremeCloud IQ using their serial numbers.

- Change OS: Select the switches in the Manage > Devices list, click Actions, and select Change OS to VOSS.

- Network Policy: Create a policy with Fabric Attach mappings for your VLANs/I-SIDs.

- Push Configuration: Perform a Complete Configuration Update. XIQ will push all the CLI commands automatically.

Extreme Portal +2

Summary Comparison

| Task | CLI Method | XIQ Method |
| --- | --- | --- |
| Effort | Manual per switch | Automated via policy |
| Speed | Instant local execution | Dependent on cloud sync |
| Visibility | Local CLI output only | Graphical Network 360 maps |
| Flexibility | High (Any command allowed) | Limited to GUI/Supplemental CLI |

Would you like to start with the CLI initialization block for the 16-port model, specifically using Port 17 for the NNI?

13 sites

Extreme Portalhttps://extreme-networks.my.site.com

Switch Operating System Selection for Universal Hardware

Jun 12, 2025 — Log in to your XIQ Administrator account at https://extremecloudiq.com . Use the wizard to select Devices. Check all the devices y...

CDWhttps://www.cdw.com

Extreme Networks ExtremeSwitching 5320-16P-2MXT-2X

Product Details * 16P-2MXT-2X. * Switch. * L3. * managed. * 16 x 10/100/1000 + 2 x 100/1000/2.5G/5G/10G (uplink) + 2 x 10 Gigabit ...

CDWhttps://www.cdw.com

Extreme Networks ExtremeSwitching 5320-16P-2MXT-2X - switch

Product Details * 16P-2MXT-2X. * Switch. * L3. * managed. * 16 x 10/100/1000 + 2 x 100/1000/2.5G/5G/10G (uplink) + 2 x 10 Gigabit ...

Show all

Yes, CLI first and I will do this, Recommendation: Use Port 17 or 18 (the 1/2.5/5/10G RJ45 ports) for the link between your two switches to keep the SFP+ slots free for future 10G fiber needs. CDW CDW +4

For your

5320-16P-2MXT-2X

home lab, here is the full CLI Initialization Block. This script assumes both switches are in VOSS (Fabric Engine) mode.

Physical Configuration Reminder

- Port 1: Internet / Home Modem

- Port 3: AP3000 Access Point

- Port 17: NNI (Fabric Link) – Recommended RJ45 Multi-Gig port

Extreme Networks

Switch 1: Gateway Switch CLI Block

Connect to your console and paste the following:

voss

# --- Global Fabric Setup ---

enable

configure terminal

router isis

system-id 0000.0000.0001

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.01

exit

router isis enable

# --- Configure NNI Link (Port 17) ---

interface gigabitEthernet 1/17

isis

isis enable

isis spbm 1

no shutdown

exit

# --- Service Provisioning (VLANs 10, 20, 30, 50, 60) ---

vlan create 10,20,30,50,60 name MGMT,Alpha,Bravo,Delta,Gamma type port-mstprstp 0

vlan i-sid 10 100010

vlan i-sid 20 100020

vlan i-sid 30 100030

vlan i-sid 50 100050

vlan i-sid 60 100060

# --- Anycast Gateway Configuration ---

interface vlan 10

ip address 10.0.10.1/24

exit

interface vlan 20

ip address 10.0.20.1/24

exit

# Repeat for other VLANs as needed...

# --- Edge Activation (AP Port 3) ---

interface gigabitEthernet 1/3

auto-sense enable

fa enable

no shutdown

exit

# --- Internet Exit (Port 1 to Modem) ---

vlan create 100 name Internet_Exit type port-mstprstp 0

vlan members add 100 1/1

interface vlan 100

ip address 192.168.1.2/24

exit

ip route 0.0.0.0 0.0.0.0 192.168.1.1 weight 1 enable

router isis

ip-shortcut

exit

save config

Use code with caution.

Switch 2: Remote Switch CLI Block

Paste this into your second switch:

voss

# --- Global Fabric Setup ---

enable

configure terminal

router isis

system-id 0000.0000.0002

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.02

exit

router isis enable

# --- Configure NNI Link (Port 17) ---

interface gigabitEthernet 1/17

isis

isis enable

isis spbm 1

no shutdown

exit

# --- Service Provisioning (Must match Switch 1) ---

vlan create 10,20,30,50,60 name MGMT,Alpha,Bravo,Delta,Gamma type port-mstprstp 0

vlan i-sid 10 100010

vlan i-sid 20 100020

vlan i-sid 30 100030

vlan i-sid 50 100050

vlan i-sid 60 100060

# --- Anycast Gateway (Identical to Switch 1) ---

interface vlan 10

ip address 10.0.10.1/24

exit

interface vlan 20

ip address 10.0.20.1/24

exit

# --- Edge Activation (AP Port 3) ---

interface gigabitEthernet 1/3

auto-sense enable

fa enable

no shutdown

exit

# --- Internet Exit (Redundant Port 1 to Modem) ---

vlan create 100 name Internet_Exit type port-mstprstp 0

vlan members add 100 1/1

interface vlan 100

ip address 192.168.1.3/24

exit

ip route 0.0.0.0 0.0.0.0 192.168.1.1 weight 1 enable

router isis

ip-shortcut

exit

save config

Use code with caution.

Verification

Once both are configured, run show isis adj. You should see a neighbor relationship on Port 17 with a state of UP.

Extreme Networks

Would you like to move on to the XIQ-based deployment steps to mirror this configuration in the cloud?

4 sites

Extreme Networkshttps://documentation.extremenetworks.com

IS-IS Configuration Parameters - VOSS User Guide

Oct 15, 2023 — * Connect a Terminal. * Changing passwords. * Configuring system identification. * Configuring the CLI Banner. * Configure the Dat...

Extreme Networkshttps://documentation.extremenetworks.com

5320-16P-2MXT-2X Switch Features

Mar 1, 2026 — The front panel of the switch includes: 1 = Mode button. 2 = System LEDs. 3 = 10/100/1000BASE-T copper RJ45 full/half duplex ports...

Extreme Networkshttps://documentation.extremenetworks.com

5320 Series Overview

Mar 1, 2026 — Model 5320-16P-2MXT-2X has a USB Type C port, but does not have a Micro-USB Type B port. After connecting to your switch, it can b...

Show all

Yes pls but before then, I want to integrate the APs?

To integrate your

AP3000s

into the VOSS Fabric, you leverage Fabric Attach (FA). This allows the APs to automatically "handshake" with the 5320 switches to request the correct VLAN/I-SID mappings for your SSIDs without you needing to manually tag those ports.

Extreme Portal +1

1. The Integration Logic

The

AP3000

acts as a Fabric Attach Client. Upon startup, it uses LLDP to discover the 5320 (the Fabric Attach Server).

Extreme Networks +1

AP3000

Integrating your

AP3000

access points with the VOSS Fabric relies on Fabric Attach (FA). This standard automates the handshake between the AP and the switch, allowing the AP to "request" its VLANs/I-SIDs.

Extreme Networks +1

1. The Fabric Attach Handshake

- Discovery: The AP3000 uses LLDP with Fabric Attach extensions to find the VOSS switch.

- Provisioning: The AP tells the switch which SSIDs it has. The switch then dynamically maps those SSIDs to the I-SIDs you defined (100010–100060).

- Auto-Sense: By enabling auto-sense on Port 3, the switch will automatically recognize the AP3000 and apply the correct configuration without you needing to manually tag the port.

Extreme Networks +4

2. Integration CLI Block (Run on both Switches)

Add these commands specifically for the APs on Port 3:

voss

# --- Global Fabric Attach (Usually on by default) ---

fa enable

# --- Configure the AP Port ---

interface gigabitEthernet 1/3

# Recognize the AP automatically

auto-sense enable

# Enable the Fabric Attach handshake

fa enable

# Trust the QoS/Voice markings from the AP

qos 802.1p-override disable

# Ensure the port is active

no shutdown

exit

Use code with caution.

3. Management VLAN Strategy

For the AP to talk to XIQ, it needs an IP address. Since you are using VLAN 10 for management, the switch must provide that to the AP:

Extreme Networks +1

- Untagged Management: If you want the AP to pick up its IP from the "Native" VLAN on the port, you can set the untagged VLAN to 10.

- Tagged Management: Most Extreme APs expect VLAN 1 (default) for initial discovery. It is often best to keep the AP port's native VLAN as 1 and let Fabric Attach handle the tagged data VLANs (20, 30, 50, 60).

Extreme Portal

4. Verification of AP Integration

Run these commands to confirm the AP and Switch are "talking":

Extreme Networks +1

| Command | Expected Result |
| --- | --- |
| show fa neighbor | You should see the AP3000 and its MAC address on Port 1/3. |
| show fa assignment | You should see your I-SIDs (e.g., 100020) mapped to VLANs on Port 1/3. |
| show lldp neighbor | Confirms basic LLDP connectivity to the AP. |

Final Step: Save Your Work

Once the AP is detected and you see the assignments in the CLI, don't forget to save:
save config

Extreme Networks

Would you like the XIQ "Network Policy" steps to ensure the AP3000 pushes the correct SSIDs to these newly created I-SIDs?

10 sites

Extreme Portalhttps://extreme-networks.my.site.com

How to configure Fabric Attach on VSP, ERS and EXOS series ...

Feb 17, 2021 — XIQ AP's support the following FA functions: Discover Fabric Attach Server upon startup. Receive management VLAN configuration fro...

Extreme Networkshttps://documentation.extremenetworks.com

Configure Fabric Attach in an SMLT - VOSS User Guide

Sep 15, 2022 — Also, the authentication key is set to the default value and the system displays the encrypted authentication key on the output. .

Extreme Networkshttps://documentation.extremenetworks.com

Auto-sense - VOSS User Guide - Extreme Networks

Dec 15, 2022 — Configure Layer 2 Trusted Auto-sense Ports. Configure EAPoL Authentication Requirements for Auto-sense Fabric Attach Clients. Conf...

Show all

yes but before... how do I provision the APs to get onto two VLANs/ Isids on each switch?

To provision the AP3000 to handle two specific VLANs/I-SIDs on each switch, you must define the mapping between the wireless SSID and the Fabric I-SID.

Since you have two VLANs per switch (plus Management), the AP needs to "trunk" these services dynamically. Here is how you do it:

1. The Strategy: Fabric Attach (FA) Mapping

You don't "push" VLANs to the AP manually. Instead, you configure the Network Policy in XIQ so that when the AP3000 boots, it tells the switch: "I am broadcasting SSID-Alpha and SSID-Bravo; please give me I-SID 100020 and 100030."

2. Configuration Steps in XIQ

- Go to Configure > Network Policies and open your policy.

- Assign SSIDs to VLANs:

  - Under Wireless Networks, create your two SSIDs (e.g., "Home_Voice" and "Home_Data").

  - In the VLAN section of each SSID, assign the corresponding VLAN ID (e.g., VLAN 20 for Voice, VLAN 30 for Data).

- Define Fabric Attach (The "Bridge"):

  - Go to Switching > Fabric Attach.

  - Create a mapping:

    - VLAN 20

→

→

I-SID 100020

    - VLAN 30

→

→

I-SID 100030

  - Repeat for the VLANs on the second switch (50 and 60).

3. CLI Verification (The Handshake)

Once you push the policy to the AP, you can see the two VLANs active on the switch port by running:

voss

show fa assignment

Use code with caution.

Expected Output:
You should see Port 1/3 listed twice—once for each service the AP is carrying:

| Port | VLAN | I-SID | Status |
| --- | --- | --- | --- |
| 1/3 | 20 | 100020 | Active |
| 1/3 | 30 | 100030 | Active |

4. What about the "Other" Switch?

Because you are using Option A (Fabric), the I-SIDs are global.

- Switch 1 port 1/3 will show assignments for I-SIDs 100020 and 100030.

- Switch 2 port 1/3 will show assignments for I-SIDs 100050 and 100060.

As long as those I-SIDs are defined in the Fabric Attach policy in XIQ, the APs will automatically request the correct pair based on which switch they are plugged into.

Would you like the XIQ screenshots or a step-by-step click guide for setting up the Fabric Attach mapping in the GUI?

Sorry. I should have clarified. How do I enable this in APs via CLI? Or these APs, when they 'call home' are given the policies by XIQ?

For your AP3000, you generally do not configure these mappings via the AP's CLI. Since these are cloud-native APs, they are designed to be "thin"—meaning they boot up, "call home" to XIQ, and pull their "intent" from the Network Policy.

However, since you want to understand the CLI side, here is how the process works for both the AP and the Switch.

1. The AP Side (Automated)

When the AP3000 calls home to XIQ, it receives a configuration that says:
"Bridge SSID 'Alpha' to VLAN 20 and use Fabric Attach to signal the switch."

The AP then automatically executes its own internal CLI (which you can see if you console into the AP):

- It enables LLDP.

- It sends Fabric Attach TLVs (Type-Length-Values) to the 5320 on Port 3.

- It "requests" the I-SIDs you mapped in the XIQ GUI.

2. The Switch Side (The CLI you MUST do)

While the AP is automated by XIQ, the 5320 Switch must be ready to receive those requests. If the switch CLI isn't set up, it will ignore the AP's "requests."

Run this on your 5320s to prepare them for the APs:

voss

# Enable the Fabric Attach Server globally

fa enable

# Configure the port where the AP is plugged in (Port 3)

interface gigabitEthernet 1/3

# Recognize the AP3000 and its requests

auto-sense enable

# Participate in the Fabric Attach handshake

fa enable

# Important: Ensure the port is not 'locked' to a specific VLAN

no shutdown

exit

Use code with caution.

3. How to Verify via CLI (On the Switch)

You can confirm the AP has successfully pulled its policy from XIQ and signaled the switch by running this on the 5320:

voss

show fa assignment

Use code with caution.

If the AP has "called home" and received its policy, you will see the I-SIDs for your two VLANs appear here as "Active".

Summary of Workflow

- Switch CLI: You paste the fa enable and auto-sense commands (as we did in the previous step).

- AP Call Home: You plug the AP into Port 3. It gets an IP, reaches XIQ, and downloads the Network Policy.

- The Handshake: The AP immediately tells the 5320: "I need VLAN 20 and 30."

- Verification: You run show fa assignment on the switch to see the magic happen.

Would you like the specific XIQ menu path to ensure your APs are correctly "pointing" to the right VLANs before they call home?

Yes pls, now let's do the AP with XIQ/ Then go into the section with deploying Switches without CLI, solely from XIQ.

In ExtremeCloud IQ (XIQ), your AP3000 access points and 5320 switches can be managed as a cohesive unit. The APs pull their "intent" from the Cloud and then negotiate with the switches via Fabric Attach (FA).

Extreme Networks +1

1. Provisioning AP3000s in XIQ

You don't configure the APs via CLI; instead, you define their behavior in a Network Policy.

Extreme Portal +1

- Define SSIDs & VLANs: Go to Configure > Wireless Networks. For each SSID (e.g., Alpha, Bravo), assign the corresponding VLAN ID (20, 30, etc.).

- Configure Fabric Attach Mapping:

  - Navigate to Configure > Network Policies and select your policy.

  - Go to the Device Template section and open the AP3000 Template.

  - Find the Wired Interface (Eth0) settings and locate the Fabric Attach section.

  - Click the + icon to map your VLAN IDs to their I-SID values (e.g., VLAN 20

→

→

I-SID 100020).

- Management VLAN: Under the Wired Interface, ensure the Management/Native VLAN is set to 10 so the AP can communicate with the switch over your MGMT network.

Extreme Networks +5

2. Deploying Switches Solely from XIQ (Zero CLI)

Universal Hardware like the

5320

is designed for "out-of-box" cloud management.

Extreme Networks +1

Step 1: Onboarding

- Log in to ExtremeCloud IQ.

- Go to Manage > Devices and click the + (Add) button

→

→

Quick Add Devices.

- Enter the Serial Numbers for your two

5320s

.

- Important: Select Fabric Engine (VOSS) as the operating system during this step.

Extreme Portal +2

Step 2: Persona Change (if currently on EXOS)

If your switches are already powered on and running EXOS:

- Select the switches in the Device List.

- Click Actions

→

→

Change OS.

- Choose VOSS (Fabric Engine) and click Save. XIQ will push the image update and reboot the switches into the new persona.

Extreme Portal +1

Step 3: Global Switch Policy

Instead of individual CLI commands, you define "Common Settings":

- In your Network Policy, go to Switching > Common Settings.

- Enable Fabric Connect (SPBM) and set your Manual Area (00.0001).

- Port Configuration: Under the Device Template for the

5320

, assign port types:

  - Port 17 (NNI): Set as a Network-to-Network Interface (IS-IS enabled).

  - Port 3 (AP): Set to Auto-Sense and enable Fabric Attach.

  - Port 1 (Modem): Set as an Access Port on your Internet VLAN (100).

Extreme Networks +4

Step 4: Push Configuration

Once your policy is ready, select both switches in the Device List, click Update, and select Perform Complete Configuration Update. XIQ will generate and push all the necessary Fabric and routing commands for you.

YouTube +1

Would you like to review the "Network 360" health metrics to confirm the Cloud has successfully pushed these Fabric settings?

17 sites

Extreme Networkshttps://documentation.extremenetworks.com

Configure Fabric Attach - ExtremeCloud IQ User Guide

Jun 15, 2023 — Before you begin. Physically connect the device to a Fabric Connect-enabled switch. To perform the following task, you will need t...

Extreme Portalhttps://extreme-networks.my.site.com

XIQ - How to create a basic Network Policy - Extreme Portal

Oct 23, 2024 — Procedure. Navigate to Configure > Network Policies > Here you have 2 options: ADD NETWORK POLICY or EXPRESS POLICY SETUP. Option ...

Extreme Networkshttps://documentation.extremenetworks.com

Configure Fabric Attach - ExtremeCloud IQ User Guide

Feb 15, 2024 — Use this task to configure a device connected to an existing Fabric Connect network. * From within the network policy, navigate to...

Show all

Yes pls

To monitor your home-lab migration and verify that ExtremeCloud IQ (XIQ) has successfully pushed the Fabric and AP settings, you use the Network 360 and Device 360 views. This is where you see the "health" of your intent-based deployment.

1. The Network 360 Plan (The Bird's-Eye View)

This view visualizes the entire topology and identifies where traffic might be bottlenecked.

- How to access: Go to ML Insights > Network 360 Monitor.

- What to look for:

  - The Topology Map: You should see a line between your two 5320s. In VOSS, XIQ identifies this as an SPB/Fabric link. If the line is green, the IS-IS adjacency is healthy [12].

  - The AP Connection: You will see your AP3000s hanging off Port 3 of each switch.

  - Health Score: XIQ aggregates data from LLDP, Fabric Attach, and throughput. A score of 90-100 means your Voice/Alpha traffic is prioritized and the "East-West" path is optimal [10].

2. Device 360 (The "Deep Dive" for Switch/AP)

If a specific VLAN isn't working, click on an individual device in the map to open its Device 360 view.

- Fabric Attach Tab: On the 5320, this tab shows exactly which I-SIDs the AP3000 has requested. It’s the visual version of the show fa assignment CLI command [9].

- Interface View: You can hover over Port 17 to see the NNI status (Fabric link) and Port 3 to see PoE power draw from the AP3000.

- VLAN/I-SID Grid: Confirms that your Alpha (20), Bravo (30), etc., mappings are active in the switch's hardware database.

3. Verification of "Option A" Resiliency

To verify the Internet Exit and Anycast Gateway we configured:

- Routing Tab: In the Switch 360 view, look at the Routing Table. You should see the 0.0.0.0/0 default route pointing to your home modem's IP.

- Client Monitoring: Connect a phone to the "Alpha" SSID. In XIQ, look at the Clientslist. It should show the phone’s IP (e.g., 10.0.20.X) and identify the 5320 as the "Anycast" gateway.

Summary Checklist for Success

- Topology Map: Are the two switches linked with a green line? (Confirms IS-IS).

- Fabric Attach assignments: Does Port 3 show two active I-SIDs? (Confirms AP Integration).

- Default Route: Does the switch see the modem? (Confirms Internet Access).

Would you like to see how to run a "Client 360" trace to watch a voice packet travel from an AP on Switch 1, across the Fabric, and out the modem on Switch 2?

Sources: [9], [10], [12]

Yes

To watch a voice packet travel across your Fabric in ExtremeCloud IQ (XIQ), you use Client 360. This tool provides a "time-travel" view of exactly how your APs and 5320s are handling your traffic.

1. Accessing Client 360

- Connect your phone or a laptop to the "Alpha" (Voice) SSID.

- In XIQ, go to Manage > Clients.

- Click on your device's MAC Address or Host Name to open the Client 360 view.

2. Following the "Voice" Path

Once the 360 view opens, look at the Topology section at the top. You will see a visual representation of the packet's journey:

- The Wireless Hop: It shows the signal strength (RSSI) between your phone and the AP3000.

- The Fabric Entry: It shows the AP3000 connected to Port 3 of your 5320.

- The Fabric Transit: If you are testing the "Option A" failover, you will see a line representing the path across the NNI (Port 17) to the other switch.

- The Exit: It shows the traffic exiting Port 1 toward your Home Modem.

3. Monitoring Quality of Service (QoS)

Since we prioritized your voice traffic (VLAN 20), check the Network Usage or SLA tab within Client 360:

- Application Visibility: Look for "Voice" or "SIP/RTP" traffic types.

- Airtime & Retries: Ensure the "Voice" traffic has low latency (typically <50ms) and minimal retries. This confirms your QoS 802.1p-override disable command on the switch is properly honoring the AP's priority tags.

4. Live Troubleshooting (Remote Sniffer)

If you suspect the packet is getting lost between switches, you can run a Remote Capture directly from XIQ:

- From the Client 360 view, click Diagnostics.

- Select Remote Sniffer.

- Choose the 5320 switch and Port 17 (NNI).

- XIQ will stream a PCAP file to your computer. When you open it in Wireshark, you will actually see the 802.1ah (MAC-in-MAC) headers wrapping your voice packets!

Summary of what you’ve built:

You now have a software-defined home lab where:

- The AP3000 manages the wireless intent.

- The 5320s manage the Fabric transport.

- XIQ provides a single pane of glass to see every "hop" from the phone to the modem.

Would you like to set up an "Alert Policy" in XIQ so you get a mobile notification if your voice latency exceeds 100ms?

Yes but first some troubleshooting. With EXOS I had issues configuring static routes from the APs back to the gateway for the VLANs. My UEs were not getting DHCP/ DNS and the port-3 were not enabled. So I had to use CLI to do it, and XIQ would not. configure vlan SW_VLAN_20 dhcp-address-range 10.20.0.100 - 10.20.0.200 configure vlan SW_VLAN_20 dhcp-options default-gateway 10.20.0.1 configure vlan SW_VLAN_20 dhcp-options dns-server primary 8.8.8.8 configure vlan SW_VLAN_20 dhcp-options dns-server secondary 8.8.4.4 enable dhcp ports 3 vlan SW_VLAN_20 configure vlan SW_VLAN_30 dhcp-address-range 10.30.0.100 - 10.30.0.200 configure vlan SW_VLAN_30 dhcp-options default-gateway 10.30.0.1 configure vlan SW_VLAN_30 dhcp-options dns-server primary 8.8.8.8 configure vlan SW_VLAN_30 dhcp-options dns-server secondary 8.8.4.4 enable dhcp ports 3 vlan SW_VLAN_30 enable ipforwarding vlan Default save configuration

That EXOS issue happened because EXOS requires explicit "per-port, per-VLAN" DHCP enablement, and XIQ's standard policy sometimes struggles to push those granular enable dhcp ports... commands correctly.

In VOSS, this is handled much more cleanly. You don't "enable DHCP on a port"; you enable the DHCP Server globally and attach it to the VLAN interface. Since Port 3 is Auto-Sense, it automatically inherits whatever services are running on the VLANs it's carrying.

VOSS CLI: Replacing your EXOS DHCP Config

Here is the VOSS equivalent to your EXOS commands. Paste this into the switch you want to act as the DHCP server (usually Switch 1):

voss

# 1. Enable DHCP Server globally

ip dhcp-server enable

# 2. Create the DHCP Pool for VLAN 20 (Alpha)

ip dhcp-server pool Alpha

network-address 10.20.0.0 255.255.255.0

range 10.20.0.100 10.20.0.200

default-router 10.20.0.1

dns-server 8.8.8.8 8.8.4.4

exit

# 3. Create the DHCP Pool for VLAN 30 (Bravo)

ip dhcp-server pool Bravo

network-address 10.30.0.0 255.255.255.0

range 10.30.0.100 10.30.0.200

default-router 10.30.0.1

dns-server 8.8.8.8 8.8.4.4

exit

# 4. Bind the pools to the VLAN interfaces

interface vlan 20

ip dhcp-server enable

exit

interface vlan 30

ip dhcp-server enable

exit

Use code with caution.

Why this fixes your "Port 3" issues:

- No Port-Binding Required: In VOSS, once ip dhcp-server enable is on the VLAN interface, any port that carries that VLAN (like Port 3 via Fabric Attach) will automatically handle DHCP requests.

- IP Forwarding is Implicit: In VOSS, once an interface has an IP, routing (forwarding) is active by default. You don't need enable ipforwarding.

- Fabric Relay: Because of your Fabric (Option A), a device on Switch 2 can get a DHCP address from the server on Switch 1 across the Fabric link without any extra config.

Troubleshooting the "Handshake"

If UEs still don't get IPs, run:

- show ip dhcp-server pool: To see if addresses are being handed out.

- show fa assignment: To ensure Port 3 actually "activated" VLAN 20/30 when the AP asked for them.

Shall we set up the Alert Policy now, or do you want to verify the DHCP bindings first?

Yes pls... also, I want an E2E trace of the route, with control plane, data plane and management plan side by side say three rows time aligned with whatever is flowing in between them from initial session setup, say my iPhone logs into the SSID to actually establishing connection and passing traffic?

This is a great way to visualize the "magic" of Fabric Connect. In VOSS, the Control Plane does the heavy lifting before the traffic even starts, which is why the Data Plane is so fast.

E2E Flow: iPhone Connection to Internet Exit

Scenario: iPhone joins SSID-Alpha (VLAN 20) on AP3000 (Switch 2), and traffic exits via Modem (Switch 1).

| Phase | Management Plane (XIQ) | Control Plane (IS-IS / FA) | Data Plane (SPB / 802.1ah) |
| --- | --- | --- | --- |
| 1. Boot & Discovery | XIQ pushes the Network Policy to AP3000 (SSID-Alpha = VLAN 20). | AP & Switch 2 perform LLDP/Fabric Attach handshake. Switch 2 "learns" AP needs VLAN 20. | No user data flowing yet. |
| 2. Fabric Prep | XIQ monitors health of the NNI (Fabric Link). | IS-IS on Switch 2 advertises: "I have I-SID 100020 active here."Switch 1 updates its forwarding table. | Link-state packets (Hellos) keep the mesh alive. |
| 3. iPhone Auth | XIQ/AP validates iPhone credentials (WPA2/3). | FA state on Switch 2 changes to Active for Port 1/3. | Association frames pass between Phone and AP. |
| 4. DHCP Request | XIQ logs "Client Connected" event. | DHCP Discover broadcast is sent. Switch 2 wraps it in a Fabric header. | MAC-in-MAC: Packet travels Switch 2 → → Switch 1 via Port 17. |
| 5. DHCP Assignment | XIQ tracks the assigned IP (10.20.0.X). | DHCP Server on Switch 1 replies. Switch 1 knows exactly where Switch 2 is via its Nick-Name. | DHCP Offer/Ack travels back through the Fabric tunnel. |
| 6. Traffic Flow | XIQ displays "Client 360" health metrics. | ARP resolved. SPBM already has the shortest path calculated. | Data Flow: Phone → → AP → → Sw2 (Encapsulate) → → Sw1 (Decapsulate) → → Modem. |
| 7. Roaming (Option) | XIQ tracks roam from AP1 to AP2. | Anycast GW means the Control Plane doesn't need to update the IP/Gateway. | Hardware simply shifts the Fabric tunnel to the new Nick-Name. |

Key Technical Takeaways for your HLD/LLD

- Management Plane (XIQ): It is "Out-of-Band." If your link to XIQ drops, the Control and Data planes keep running locally. Your lab won't break.

- Control Plane (IS-IS): This is "Silent." Unlike EXOS, which constantly floods ARP/Broadcasts to find paths, VOSS calculates everything in the background using the link-state database.

- Data Plane (MAC-in-MAC): This is "Stealth." The core of your network (the NNI link) never sees the iPhone's MAC address; it only sees the "Outer" Switch MAC addresses. This is why it scales so well.

Troubleshooting Tip for your Lab

If the iPhone stays in "Obtaining IP Address" (the issue you had with EXOS):

- Check Control Plane: show fa assignment (Is the VLAN active on the port?)

- Check Data Plane: show isis spbm i-sid (Does Switch 1 know Switch 2 has that I-SID?)

- Check DHCP: show ip dhcp-server binding (Did Switch 1 actually give out the IP?)

Shall we proceed with setting up the "Alert Policy" in XIQ so you get a ping if that IS-IS Control Plane heartbeat ever skips a beat?

Yes but first, can you animate the three planes above in a sequence that is executable / so we can stop and start and see the messaging etc. that flows?

While I cannot generate a playable video file (MP4/GIF) directly in this chat, I can provide a Step-by-Step Logic Flow that functions like an "Action Script." You can use this to walk through the process on your lab equipment, stopping at each point to run the CLI commands to see the "frozen" state of the messaging.

The "Anatomy of a Connection" (Step-by-Step)

Stage 1: The "Ready" State (Management Plane)

- Action: Push your Network Policy from XIQ to the AP and Switches.

- The Message: XIQ sends an SSH/HTTPS payload containing the "Intent" (SSIDs, VLAN 20, I-SID 100020).

- Pause & Verify (CLI):

  - Switch: show application iqagent status (Confirm "Connected").

  - AP: In XIQ, check that the config audit is Green.

Stage 2: The "Handshake" (Control Plane)

- Action: Plug the AP3000 into Port 3.

- The Message (LLDP-FA): The AP sends an LLDP packet with a "Fabric Attach" TLV. It says: "I am an AP; I need I-SID 100020 for my SSID."

- Pause & Verify (CLI):

  - Switch: show fa neighbor (You’ll see the AP).

  - Switch: show fa assignment (You’ll see the I-SID status as Pending/Active).

Stage 3: The "Tunnel" (Control Plane)

- Action: IS-IS processes the new service.

- The Message (IS-IS LSP): Switch 2 sends a "Link State PDU" to Switch 1. It says: "I now host I-SID 100020. Send all traffic for this service to my Nick-Name (0.00.02)."

- Pause & Verify (CLI):

  - Switch 1: show isis spbm i-sid (Confirm Switch 1 now "sees" the service on Switch 2).

Stage 4: The "First Data" (Data Plane)

- Action: Connect your iPhone to the SSID.

- The Message (DHCP Discover): The iPhone sends a broadcast.

- The Transformation:

  - AP: Tags it VLAN 20.

  - Switch 2: Wraps it in a MAC-in-MAC header (Outer Source: Sw2, Outer Dest: Sw1).

  - Switch 1: Strips the header and hands it to the DHCP Server process.

- Pause & Verify (CLI):

  - Switch 1: show ip dhcp-server binding (See the iPhone’s MAC address).

Stage 5: "Active Flow" (Data Plane)

- Action: iPhone streams a video or makes a Voice call.

- The Message (Standard Traffic): Bidirectional flow using the shortest path.

- Pause & Verify (CLI):

  - Switch: show isis spbm unicast-tree (See exactly which physical path the data is taking).

How to "Animate" this in your Lab:

To see this in slow motion, you can use the VOSS Trace tool to capture the messages as they happen:

- Start a Trace on Switch 1:
trace level isis 3 (This shows the IS-IS Control Plane messages in real-time).

- Start a Trace on the AP Port:
trace level fabric-attach 3 (This shows the FA handshake as you plug in the AP).

- Connect the iPhone and watch the CLI scroll with the exact messages mapped in the table above.

Ready to set up that Alert Policy now so you're notified the next time Stage 2 or 3 fails?

Yes give me the alert policy

To ensure your home lab is "production-grade," you need an Alert Policy in ExtremeCloud IQ (XIQ) that specifically monitors the health of the Fabric Control Plane and the AP integration.

1. Creating the Alert Policy in XIQ

- Go to Monitor > Alarms > Alarm Settings.

- Click + Add to create a new Alert Policy.

- Name it: Home_Lab_Fabric_Alerts.

2. Critical Alarms to Enable

Check the boxes for these specific events to monitor your 3-Plane architecture:

| Alarm Category | Event Name | Why it matters |
| --- | --- | --- |
| System | IS-IS Adjacency Change | Notifies you instantly if the "East-West" link (Port 17) between switches fails. |
| System | Device Neighbor Loss | Alerts you if the AP3000 stops communicating with the switch via LLDP. |
| Network | Fabric Attach Assignment Failed | Alerts you if the AP requests a VLAN (like Voice) but the switch rejects it. |
| Device | Device Disconnected | Notifies you if a switch or AP loses its "Management Plane" connection to XIQ. |
| Client | DHCP Failure | Alerts you if your iPhone fails to get an IP (the issue you had with EXOS). |

3. Setting Up Notifications

- In the same policy, scroll to Notifications.

- Add your Email Address or integrate with Webhooks (if you use Slack/Discord for lab alerts).

- Set the Severity for "IS-IS Adjacency Down" to Critical.

4. CLI-Based Local Monitoring (The "Syslog" backup)

Since you like the CLI, you can also tell the switches to send a "Log Trap" to your management console if the Fabric fails:

voss

# On both 5320s

logging remote address <Your_PC_IP>

logging remote control enable

logging remote level informational

# Specifically monitor IS-IS (Control Plane) and FA

logging remote facility local0

Use code with caution.

5. Final "Success" Verification

To test your new Alert Policy:

- Unplug the Port 17 (NNI) cable.

- Wait ~10 seconds.

- Check your email or the XIQ Alarms dashboard. You should see a Critical Alarm: "IS-IS Adjacency state changed to DOWN on unit 1 port 17."

Your VOSS Home-Lab is now fully designed, deployed, and monitored!

Do you want to explore ExtremeCloud IQ "Network Scorecard" next to see how your Wi-Fi performance compares to industry standards, or are you ready to start the physical migration?

Yes pls, although I have not yet deployed... the lab in VOSS yet, I want to 'simulate' the E2E work, like they did in the space-station before actual launch. Can you create such an environnemtn?

Since we can’t physically touch the hardware yet, we will build a Digital Twin Simulationof your flight plan. This uses a "Pre-Flight Checklist" and a Simulated CLI walkthroughto validate the logic of your Control, Data, and Management planes.

The "Pre-Launch" Simulation Environment

1. The Flight Path (E2E Connectivity)

- The Ship: Two 5320-16P switches + two AP3000s.

- The Mission: Stretch VLAN 20 (Voice) and 30 (Data) across a Fabric without manual trunks.

- The Rocket Fuel: IS-IS (Control Plane) and 802.1ah (Data Plane).

2. Simulated "Ignition" (Boot Sequence)

We will simulate the CLI responses you should see at each stage.

Test Case A: IS-IS Adjacency (The Backbone)

- Simulated Action: You plug the cable into Port 17 on both switches.

- Command: show isis adj

- Expected Output (The "Green Light"):

text

INTERFACE  NEIGHBOR         STATE   LEVEL  METRIC  PRIORITY

----------------------------------------------------------

1/17       0.00.02          UP      L1     10      64

Use code with caution.

If State is anything other than UP, mission scrubbed (check Ethertype 0x8100).

Test Case B: Fabric Attach (The AP Handshake)

- Simulated Action: AP3000 boots and "calls home" to XIQ. It requests VLAN 20.

- Command: show fa assignment

- Expected Output:

text

PORT    VLAN    I-SID      STATE      TYPE

-------------------------------------------

1/3     20      100020     ACTIVE     DYNAMIC

Use code with caution.

If State is 'Pending', check if 'auto-sense' is enabled on the switch port.

Test Case C: Anycast Gateway (The Resiliency)

- Simulated Action: iPhone on Switch 2 pings 10.20.0.1. You unplug the NNI link.

- Expected Behavior: The iPhone continues to ping because Switch 2 sees it has a local instance of that gateway and its own path to the Modem (Port 1).

3. The "Stress Test" (Failure Simulation)

In our simulation, we trigger a "Critical Failure" on Switch 1's Modem Link.

| Failure Event | Plane Affected | System Response |
| --- | --- | --- |
| Port 1 Link Lost | Data Plane | Switch 1 detects its local route to 0.0.0.0 is gone. |
| Route Recalculation | Control Plane | IS-IS updates. Switch 1 sees Switch 2 still has a path to the Modem. |
| Reroute | Data Plane | All Switch 1 users now travel across Port 17 → → Switch 2 → → Modem. |
| Notification | Management | XIQ Alarms logs: "Static Route Updated - Next Hop shifted to 0.00.02." |

4. Running the "Dry Run" Script

You can "execute" this simulation by reviewing your CLI blocks one last time against the logic:

- Do the I-SIDs match exactly? (100020 on both?)

- Is the Nick-name unique? (0.00.01 vs 0.00.02?)

- Is DHCP bound to the VLAN?

Would you like me to generate a "Final Pre-Flight Checklist" PDF-style summary that you can print and have next to you during the actual launch?

Yes pls, I want to give it to NotebookLLM or Claude etc. to create a digital twin, is that possible?

Yes, that is absolutely possible. To build a high-fidelity Digital Twin, you need to feed those models a structured "System Prompt" or "Context File" that defines the state machine, hardware constraints, and protocol behaviors of VOSS and XIQ.

Copy the text below. It is formatted specifically as a "System Definition & State Map"that Claude or NotebookLM can use to simulate your environment.

System Definition: Extreme Networks VOSS Fabric Digital Twin

1. Hardware Specification (The Nodes)

- Node_01 (Switch-1): Extreme 5320-16P-2MXT-2X. Nickname: 0.00.01. System-ID: 0000.0000.0001.

- Node_02 (Switch-2): Extreme 5320-16P-2MXT-2X. Nickname: 0.00.02. System-ID: 0000.0000.0002.

- Node_03/04 (APs): AP3000 Series (Fabric Attach Clients). Connected to Port 1/3 on both switches.

- Node_05 (Gateway): Home Modem (IP 192.168.1.1). Connected to Port 1/1 on both switches.

2. Protocol Logic (The Control Plane)

- Fabric Connect: IEEE 802.1aq (SPB) / RFC 6329 (IS-IS).

- Backbone (NNI): Port 1/17 on both nodes. Ethertype: 0x8100. Area: 00.0001.

- Service Mapping (I-SIDs):

  - VLAN 10 (MGMT) -> I-SID 100010

  - VLAN 20 (Alpha/Voice) -> I-SID 100020

  - VLAN 30 (Bravo/Data) -> I-SID 100030

  - VLAN 50 (Delta) -> I-SID 100050

  - VLAN 60 (Gamma) -> I-SID 100060

3. Behavioral Logic (The Digital Twin Rules)

- Anycast Gateway Rule: Both Switches respond to 10.0.X.1. If an NNI link fails, the local switch handles routing immediately (Sub-second convergence).

- Fabric Attach Rule: If Port 1/3 receives an LLDP-FA TLV from an AP3000, it must dynamically provision the assigned I-SIDs.

- DHCP Rule: Switch-1 is the authoritative DHCP server for all VLANs. Requests from Switch-2 must be encapsulated in MAC-in-MAC (I-SID tunnel) to reach the server.

- Management Rule: All nodes must maintain a persistent heartbeat to ://extremecloudiq.com (XIQ).

4. Operational State Commands (Simulation Triggers)

- show isis adj: Return "UP" if Port 1/17 parameters match.

- show fa assignment: Return "Active" for VLAN 20/30 if AP3000 is detected on Port 1/3.

- show ip route: Show 0.0.0.0 via 192.168.1.1. If Port 1/1 is DOWN, show 0.0.0.0 via IS-IS Nickname.

Instructions for your AI Digital Twin:

- Upload the text above to Claude or NotebookLM.

- Command it: "Act as a network simulator for this specific Extreme Networks VOSS lab. I will provide CLI commands or physical 'actions' (like unplugging a cable), and you will describe the resulting state changes in the Management, Control, and Data planes."

Would you like me to add the "Failure Scenarios" to this definition so the Digital Twin knows how to react when things go wrong?

Yes pls

To make your Digital Twin truly "intelligent," you need to define the State Transitions for when things break. Adding these logic gates allows Claude or NotebookLM to simulate the sub-second recovery of Fabric Connect.

Copy and append this to your System Definition file:

5. Failure & Recovery Logic (State Transitions)

Scenario A: NNI Link Failure (Port 1/17)

- Trigger: Physical link between Switch-1 and Switch-2 is disconnected.

- Control Plane Response: IS-IS Adjacency moves to DOWN. Link State Database (LSDB) is updated on both nodes.

- Data Plane Response:

  - East-West Traffic: Traffic between AP-1 and AP-2 is severed.

  - North-South Traffic: Anycast Gateway logic ensures Switch-2 traffic exits via its local Modem (Port 1) instead of traversing the Fabric.

- Management Plane Response: XIQ triggers a "Critical" alarm: IS-IS Adjacency Lost.

Scenario B: Modem Link Failure (Port 1/1 on Switch-1)

- Trigger: Cable to Home Modem on Switch-1 is unplugged.

- Control Plane Response: Static route 0.0.0.0 is removed from Switch-1's routing table.

- Data Plane Response:

  - The Reroute: Switch-1 sees the "IP Shortcut" advertised by Switch-2.

  - The Path: Traffic from Switch-1 is encapsulated in I-SID 100100 (Internet VSN) and sent across Port 17

→

→

Switch-2

→

→

Modem.

- Simulation Check: Ping to 8.8.8.8 from a device on Switch-1 should drop exactly 0-1 packets.

Scenario C: AP3000 Fabric Attach Timeout

- Trigger: AP3000 stops sending LLDP heartbeats.

- Control Plane Response: Switch-1/2 timer expires (90 seconds). Fabric Attach state moves to INACTIVE.

- Data Plane Response: The switch dynamically removes VLAN 20/30 from Port 1/3 to prevent unauthorized access.

- Management Plane Response: XIQ displays "Device Disconnected" for the AP3000.

Scenario D: DHCP Server Exhaustion

- Trigger: Number of UEs (User Equipment) exceeds the 100-200 range.

- Behavior: Switch-1 (DHCP Server) logs a "Pool Exhaustion" error.

- Result: New iPhone connections fail at the "Obtaining IP" stage. Data plane remains UP, but Control Plane (DHCP) fails.

How to use this with your Digital Twin (Claude/NotebookLM):

Once you've uploaded the full definition, try asking the AI these "Pre-Flight" questions:

- "I just pulled the cable on Port 17. Walk me through the ISIS adjacency state and how my Voice traffic on VLAN 20 gets to the internet."

- "If Switch-1 is my DHCP server, and the NNI link is down, can an iPhone connected to Switch-2 still get an IP address?" (The Twin should answer: No, because the Fabric tunnel to the server is broken).

Would you like me to create a "Recovery Script" for the Digital Twin so it can tell you the exact CLI commands to fix these issues if they occur?

Yes pls

To complete your Digital Twin definition, here is the Recovery & Diagnostic Script. This provides the AI (and you) with a decision tree to move from a "Failed" state back to "Mission Success."

6. Recovery & Diagnostic Decision Tree (The "Fix-It" Logic)

Level 1: Fabric Core Recovery (NNI / IS-IS)

Symptoms: show isis adj is empty; Anycast routing is inconsistent.

- AI Simulation Logic:

  - Check Physical: show interface gigabitEthernet 1/17 (Ensure state is up/up).

  - Check Protocol: show spbm (Verify ethertype is 0x8100 on both).

  - The "Pulse" Command: router isis enable (Ensure the global process wasn't accidentally disabled).

- Recovery Action: If Ethertypes match and ports are up, run default interface gigabitEthernet 1/17 followed by the NNI CLI block to reset the handshake.

Level 2: AP Integration Recovery (Fabric Attach)

Symptoms: AP is powered on but SSIDs aren't passing traffic; show fa assignment is blank.

- AI Simulation Logic:

  - Verify Handshake: show fa neighbor (If empty, LLDP is blocked).

  - Check Auto-Sense: show interface gigabitEthernet 1/3 verbose (Ensure auto-sense is enabled).

  - VLAN Presence: show vlan i-sid (Ensure the I-SID exists; the switch won't grant a request for a non-existent service).

- Recovery Action: Toggle the handshake: interface gig 1/3, no fa enable, fa enable. This forces a re-sync with the AP.

Level 3: Internet/DHCP Recovery

Symptoms: Users have "No Internet" or "Self-assigned IP (169.254.x.x)".

- AI Simulation Logic:

  - DHCP Check: show ip dhcp-server summary (Verify server is enabled).

  - Route Check: show ip route (Verify 0.0.0.0 exists via the Modem IP).

  - Modem Link: ping 192.168.1.1 (Confirms Switch-to-Modem Layer 2 connectivity).

- Recovery Action: If the route is missing, re-apply: ip route 0.0.0.0 0.0.0.0 <Modem_IP> weight 1 enable.

How to use this with NotebookLM / Claude:

You can now ask the model:

- "The Digital Twin shows 'No Internet' on Switch-2. Based on the Recovery Script, what is the first command I should run to see if the Fabric is backhauling the route?" (The Twin should suggest show ip route to look for the IP Shortcut).

Final Launch Recommendation:

Now that your Digital Twin is fully defined (Hardware, Protocols, Failure Scenarios, and Recovery), you are ready for the physical migration.

Pro-Tip: Before you run the Change OS command on your live 5320s, copy-paste your final VOSS CLI script into a notepad so you can "Stage" the commands as the switches reboot into their new persona.

Would you like me to consolidate this entire E2E plan into a single "Master Lab Guide" text block for your final record?

Yes pls

This Master Lab Guide consolidates your E2E HLD, LLD, and Digital Twin logic into a single reference for your migration from EXOS to VOSS on the 5320-16P switches.

MASTER LAB GUIDE: EXOS TO VOSS FABRIC MIGRATION

1. ARCHITECTURE (HLD)

- Topology: Two 5320-16P switches in a "Fabric Connect" mesh (Option A).

- Control Plane: SPB (802.1aq) using IS-IS.

- Data Plane: MAC-in-MAC (802.1ah) encapsulation.

- Edge: AP3000s using Fabric Attach (FA) for dynamic VLAN/I-SID provisioning.

- Resiliency: Anycast Gateways and IP Shortcuts for sub-second convergence.

2. CONFIGURATION DETAILS (LLD)

Physical Mapping

- Port 1: Internet / Home Modem (192.168.1.1)

- Port 3: AP3000 Access Point (Fabric Attach)

- Port 17: NNI / Fabric Backbone (Multi-Gig RJ45)

Service Mapping

| Service | VLAN | I-SID | IP Gateway |
| --- | --- | --- | --- |
| Management | 10 | 100010 | 10.0.10.1 |
| Alpha (Voice) | 20 | 100020 | 10.0.20.1 |
| Bravo | 30 | 100030 | 10.0.30.1 |
| Delta | 50 | 100050 | 10.0.50.1 |
| Gamma | 60 | 100060 | 10.0.60.1 |

3. PRE-FLIGHT CLI SCRIPT (READY-TO-PASTE)

Switch 1 (Primary)

voss

# --- FABRIC CORE ---

router isis

system-id 0000.0000.0001

manual-area 00.0001

exit

spbm 1

spbm ethertype 0x8100

spbm nick-name 0.00.01

exit

router isis enable

# --- NNI & EDGE ---

interface gigabitEthernet 1/17

isis enable

isis spbm 1

exit

interface gigabitEthernet 1/3

auto-sense enable

fa enable

exit

# --- SERVICES & DHCP ---

vlan create 10,20,30,50,60 type port-mstprstp 0

vlan i-sid 10 100010

vlan i-sid 20 100020

vlan i-sid 30 100030

vlan i-sid 50 100050

vlan i-sid 60 100060

interface vlan 10,20,30,50,60

ip dhcp-server enable

exit

# --- INTERNET EXIT ---

vlan create 100

vlan members add 100 1/1

interface vlan 100

ip address 192.168.1.2/24

exit

ip route 0.0.0.0 0.0.0.0 192.168.1.1 weight 1 enable

router isis

ip-shortcut

exit

save config

Use code with caution.

(Repeat global/service blocks for Switch 2 using system-id ...0002 and nick-name 0.00.02)

4. DIGITAL TWIN SIMULATION LOGIC

- Management Plane: XIQ pushes "Intent." Use show application iqagent status to verify.

- Control Plane: IS-IS builds the map. Use show isis adj and show fa assignmentto verify.

- Data Plane: Traffic is tunneled. Use show isis spbm i-sid to verify East-West flow.

- Failure Recovery: If Port 17 fails, traffic stays local via Anycast. If Port 1 fails, traffic reroutes via the NNI via IP Shortcuts.

5. MISSION SUCCESS VERIFICATION

- Fabric Check: show isis adj (State: UP).

- AP Check: show fa assignment (State: ACTIVE).

- Client Check: Connect iPhone, verify IP from Switch 1, browse internet.

- Convergence Test: Unplug NNI; verify <1s drop in Voice/Ping.

Your VOSS migration plan is now complete. Do you have any final questions before you execute the Persona Change in XIQ?

Not right now, but perhaps a diagram for the E2E architecture and a cheat-sheet of the major component-wise updates from EXOS to VOSS.

Since you are preparing for a full migration, here is the E2E architecture visualization and a high-level cheat sheet of the fundamental updates when moving from EXOS to VOSS on your 5320-16P units.

E2E Home-Lab Architecture (VOSS Option A)

This diagram illustrates the "East-West" Fabric link and the "North-South" redundant internet paths using your specific port assignments.

- NNI (Fabric Link): A high-speed multi-gig path (Port 17) that "stretches" your VLANs without manual tagging.

- Fabric Attach (FA): The AP3000 (Port 3) uses LLDP-FA to dynamically request its VLAN/I-SID mappings.

- Anycast Exit: Both switches (Port 1) talk to the Home Modem; your gateway IP (10.0.X.1) is active on both simultaneously.

Cheat-Sheet: EXOS to VOSS Component Updates

When you move from EXOS (Switch Engine) to VOSS (Fabric Engine), the "vocabulary" and operational logic of your network shift from static manual paths to automated service-based paths.

| Component | EXOS (Switch Engine) Logic | VOSS (Fabric Engine) Logic |
| --- | --- | --- |
| Control Plane | STP / MLAG: Blocks redundant links to prevent loops. | SPB / IS-IS: Shortest path mesh; all links are active and loop-free. |
| Trunking | VLAN Tagging: Must manually tag every VLAN (10, 20, 30) on every uplink port. | NNI (Network-to-Network Interface): Enable IS-IS once; all I-SIDs flow automatically. |
| Service Mapping | VLAN ID: Local only; requires manual consistency across the network. | I-SID (24-bit): Global service identifier that bridges traffic across the Fabric. |
| AP Integration | Static Configuration: Manually set the port as "Tagged" for specific VLANs. | Fabric Attach (FA): AP "requests" the VLANs it needs; switch provisions them on-demand. |
| Redundancy | VRRP: Active/Standby VIP for the gateway. | Anycast Gateway: The same IP is active on all switches; nearest switch responds. |
| Port Logic | Fixed Type: Port is defined as Access or Trunk. | Auto-Sense: Port "sniffs" the device (AP, Phone, or Switch) and configures itself. |
| Configuration | Saving: Uses save configuration (ASCII/XML). | Saving: Uses save config (binary-based primary configuration). |

Quick Tips for the 5320-16P

- OS Change: During reboot, press the space bar at the Boot Menu and select "Change the switch OS to VOSS".

- Management: VOSS uses a "Segmented Management Instance" for in-band or out-of-band control, ensuring your management traffic doesn't mix with user data.

Are you ready for the final "Ignition" command in XIQ to begin the persona change, or shall we verify your I-SID numbering one last time?

15 sites

Extreme Networkshttps://www.extremenetworks.com

5320 Series - Extreme Networks

16-port 5320 models operate silently at temperatures up to 35°C/45°C (95°F/113°F), depending on the specific model. This makes the...

Extreme Portalhttps://extreme-networks.my.site.com

Fabric Engine Best Practice - Autosense Ports - Extreme Portal

Nov 3, 2025 — Details * Fabric to the edge. Extreme's Fabric Edge solution reduces the number of network protocols by replacing the multi-chassi...

Extreme Networkshttps://documentation.extremenetworks.com

Fabric Attach Deployment Scenarios - VOSS User Guide

Aug 15, 2022 — Fabric Attach is typically deployed in the access layer(s) of a Fabric Connect network. Fabric Attach, when used with a Fabric Con...

Show all

Before that, I want to SAVE all of my EXOS configs in both the switches and APs.

To ensure a safe transition to VOSS, you must save and export your current configurations off the devices. Changing the operating system on the 5320s is a destructive process that will wipe all local configuration files, logs, and events.

Extreme Portal +2

1. Backing Up 5320 Switches (EXOS)

For your switches, you should save both a binary database file and a human-readable CLI script to an external server (like a TFTP server).

Extreme Portal +1

- Save locally first:
save (Enter y at the prompt).

- Export as a CLI Script (.xsf): This creates a text file identical to the output of show configuration.
upload configuration <TFTP_SERVER_IP> switch1_backup.xsf vr VR-Default.

- Export as a Binary/XML Database (.cfg):
tftp put <TFTP_SERVER_IP> primary.cfg vr VR-Default.

Extreme Portal +4

Alternatively, if managed by XIQ: You can go to Configure > Devices > Switches, select your switch, and use Create Backup to store a copy in the cloud.

Extreme Networks +1

2. Backing Up AP3000 Access Points

Since APs are cloud-managed, their primary "configuration" is the Network Policy in ExtremeCloud IQ.

Extreme Portal

- Export via Configuration Audit: There is no standard "export" button for AP configurations in XIQ.

  - Navigate to Manage > Devices in ExtremeCloud IQ.

  - Locate the green "Configuration Audit" icon next to your AP.

  - Click the "Complete" tab to see the full CLI.

  - Copy and paste this text into a local file (e.g., AP1_config_backup.txt).

- Archive the Network Policy:

  - Go to Configure > Network Policies.

  - Select your policy and use the Clone option to create a static copy of your working EXOS policy before you begin modifying it for VOSS.

Extreme Portal +1

Summary Backup Checklist

| Device | Backup Type | Command / Action |
| --- | --- | --- |
| 5320 Switch | Human-Readable | upload configuration <IP> <filename>.xsf |
| 5320 Switch | Binary/DB | tftp put <IP> primary.cfg |
| AP3000 | CLI Transcript | XIQ -> Device -> Configuration Audit -> Complete (Copy/Paste) |
| Lab Policy | Cloud Archive | XIQ -> Network Policies -> Clone |

Do you have your TFTP or SCP server ready to receive these files, or would you like to see how to save them to a local USB drive on the switch instead?

Extreme Networks +1

9 sites

Extreme Portalhttps://extreme-networks.my.site.com

How To: How to Backup and Restore a Switch Engine (EXOS ...

Mar 31, 2026 — Procedure. There are two possible methods for this. The first method involves generating a XOS Scripting File (. xsf) from the con...

Extreme Portalhttps://extreme-networks.my.site.com

Is there a method to Export and/or Backup AP Configurations ...

Mar 23, 2023 — Answer. There is no method to export the onboarded APs via ExtremeCloud IQ. If this is a requirement, administrator would need to ...

Extreme Networkshttps://documentation.extremenetworks.com

ExtremeXOS Quick Guide - Product Documentation

Nov 15, 2015 — save config filename Writes the current config to a new file name, and then asks if you want to make the new file the default data...

Show all

USB would be good. I also have XIQ. So both.

Saving your backups to both a USB drive and ExtremeCloud IQ (XIQ) provides the best "safety net" before the VOSS conversion. Changing the OS will reformat the 5320 switches' storage, so having an external copy is critical.

Extreme Networks +1

1. Backup to USB (EXOS CLI)

EXOS supports FAT32-formatted USB drives. The mount point for these is /usr/local/ext.

Extreme Portal +1

- Format Check: Ensure your USB is FAT32. NTFS is not supported.

- Enable USB: If the port isn't active, run enable switch usb and reboot to activate it.

- Save the Binary Config:

exos

save

cp /usr/local/cfg/primary.cfg /usr/local/ext/primary_backup.cfg

Use code with caution.

- Save the Human-Readable Script (.xsf):
This creates a text file of every CLI command currently in use, which is easier to read later.

exos

save configuration as-script switch_config_backup.xsf

cp /usr/local/cfg/switch_config_backup.xsf /usr/local/ext/switch_config_backup.xsf

Use code with caution.

Extreme Portal +4

2. Backup to ExtremeCloud IQ (XIQ)

XIQ automatically attempts to store a configuration record when a device is connected, but you can force a manual "Golden" backup for safety.

Extreme Networks

- Switch Backup:

  - In ExtremeCloud IQ, go to Manage > Devices.

  - Select your 5320 Switch.

  - Select Backups (from the top bar or right-click menu) and click Create Backup.

  - Verify the backup appears in the Backups > View list.

- AP Configuration Record:
Since the AP3000's logic lives in the Cloud policy, you should archive the current state.

  - Go to Configure > Network Policies.

  - Select your current EXOS policy and click Clone. Name it EXOS_Pre_Migration_Backup to preserve your old SSID and VLAN settings.

  - For a hard copy of the AP's specific settings, click on the Configuration Audit icon next to the AP in the Devices list and copy the text from the Complete tab to a local document.

Extreme Networks +3

Summary Checklist

| Goal | Media | Key Command / Location |
| --- | --- | --- |
| Readable Script | USB | save configuration as-script then cp to /usr/local/ext |
| Binary Backup | USB | save then cp primary.cfg to /usr/local/ext |
| Cloud Backup | XIQ | Manage > Devices > Select Switch > Create Backup |
| Policy Archive | XIQ | Configure > Network Policies > Clone |

Would you like to verify the contents of the USB drive directly from the switch CLI to ensure the files were written correctly?

12 sites

Extreme Networkshttps://documentation.extremenetworks.com

Switch Configuration Backup Files - Product Documentation

Oct 27, 2023 — Switch Configuration Backup Files. When a switch is changed to CLI-mode, ExtremeCloud IQ Controller automatically creates a backup...

Extreme Networkshttps://documentation.extremenetworks.com

Managing the Configuration File - Product Documentation

Mar 1, 2026 — ExtremeXOS configuration files are saved in Extensible Markup Language (XML) format. Use the show configuration command to view on...

Extreme Portalhttps://extreme-networks.my.site.com

How to Backup running config to USB - Extreme Portal

Jun 27, 2025 — 1) Insert the USB drive into the USB port of the switch. 2) Ensure the USB drive is formatted as FAT or FAT32, as EXOS does not su...

Show all

Yes

To verify that your EXOS configuration files were correctly saved to the USB drive, you can use the following ExtremeXOS (EXOS) CLI commands.

1. Verify USB Mount Status

First, confirm the switch has correctly mounted the USB drive. The command depends on your EXOS version:

- EXOS 31.1 or Newer: Use show switch mounts.

- EXOS 30.7 or Older: Use show memorycard.

2. List Files on the USB

The USB drive is mapped to the directory /usr/local/ext. To see a list of files saved there, run:

Extreme Networks +1

exos

ls /usr/local/ext

Use code with caution.

This ls command will display the filename, file size, and the date and time it was last modified. You should see your .cfg and .xsf files listed here.

Extreme Networks

3. (Optional) Check File Content

If you want to view a few lines of the readable script (.xsf) to be 100% sure it isn't empty, use the show command:

exos

show file /usr/local/ext/your_backup_filename.xsf

Use code with caution.

Pro-Tip for Verification

If you need to check the exact size of the external memory or troubleshoot mount issues:

- Check Media Details: Use show sys-info usb.

- Format Check: If the drive isn't detected, ensure it is FAT32 formatted. You can also use dos-format /usb from the CLI if needed.

Extreme Portal +1

Would you like the final commands to safely "unmount" the USB before you physically remove it to prevent data corruption?

7 sites

Extreme Portalhttps://extreme-networks.my.site.com

How To: Upgrade Switch Engine (EXOS) via USB

Aug 8, 2025 — Procedure * Format the USB drive to a FAT or FAT32 filesystem. * Place an EXOS / Switch Engine image in the root directory of the ...

Extreme Networkshttps://documentation.extremenetworks.com

Displaying Files on the Switch - ExtremeXOS® User Guide

Sep 15, 2022 — Displaying Files on the Switch. You can display a list of the configuration, policy, or if configured, core dump files stored on y...

avayaucblog.comhttps://avayaucblog.com

Software Upgrade using USB on EXOS - explorerob's Blog

Sep 7, 2022 — #EXOS 30.7 or Older show memorycard ls /usr/local/ext download image memorycard .xos #EXOS 31.1 or Newer show switch mounts ls /us...

Show all

Yes

To prevent file corruption, you must stop all active processes from accessing the drive before pulling it out.

1. Unmount the USB

Run the following command to safely eject the drive:

exos

unmount /usr/local/ext

Use code with caution.

2. Verify it is safe to remove

Run the mount check again to ensure the directory is no longer active:

- EXOS 31.1+: show switch mounts

- Older versions: show memorycard

If the /usr/local/ext entry is gone, you can physically remove the USB.

3. Verification Tip

If the switch returns an error saying "Device is busy," it usually means you still have a file open (like if you just ran show file or ls). Simply wait a few seconds and try the unmount command again.

Now that your EXOS backups are safely on the USB and in XIQ, are you ready to execute the Change OS command in XIQ to begin the VOSS transformation?